[image: image128.wmf]Peripheral Circuits

PLL

HK ADC

Microprocessor Bus

Address/Data

General

-

Purpose Interface Bus

JTAG Interfaces

Rx

ADCs

RF

Interface

TX

DACs

AUX

SBI

SBI

Vocoder

EVRC

13

k

MP4

MIDI

QDSP4000

Integrated

CODEC

MODE Select

Interface

ETM

gpsOne

Processor

UART1

UART2

UART3/USB

ARM9

TDMI

Memory

Controller

General

-

Purpose

Interface

Camera module

(1.3M C

-

MOS)

PC

Connectivity

Test/Debug

System

Digital Test Bus

JTAG

Keypad input

RFT6100

RFR6000

MSM6500

Mobile Station Modem

VCTCXO

19.2

MHz

1

2

3

4

5

6

7

8

9

*

#

0

POWER

Amplifier

LCD

Memory

NAND Flash(512Mb)

LP

-

SDRAM(256Mb)

EAR

External

EAR/MIC

KEYPAD, BEZEL,

SIIDEKEY

MIC

Audio AMP

Data Comm

DM

USB

USB

Transceiver

BATTERY

HDET

INNER CHG

+2.9

V_Rx_LDO

DFM

Processor

CDMA

Processor

RF Controls

2

Speaker

LDO

CAMIF

Bluetooth

Baseband

Processor

B/T Chip

Antenna

Bluetooth RF

2

Peripheral Circuits

PLL

HK ADC

Microprocessor Bus

Address/Data

General

-

Purpose Interface Bus

JTAG Interfaces

Rx

ADCs

RF

Interface

TX

DACs

AUX

SBI

SBI

Vocoder

EVRC

13

k

MP4

MIDI

QDSP4000

Integrated

CODEC

MODE Select

Interface

ETM

gpsOne

Processor

UART1

UART2

UART3/USB

ARM9

TDMI

Memory

Controller

General

-

Purpose

Interface

Camera module

(1.3M C

-

MOS)

PC

Connectivity

Test/Debug

System

Digital Test Bus

JTAG

Keypad input

RFT6150

RFR6500

MSM6500

Mobile Station Modem

VCTCXO

19.2

MHz

1

2

3

4

5

6

7

8

9

*

#

0

POWER

Amplifier

LCD

Memory

NAND Flash(512Mb)

LP

-

SDRAM(512Mb)

EAR

External

EAR/MIC

KEYPAD, BEZEL,

SIIDEKEY

MIC

Audio AMP

Data Comm

DM

USB

USB

Transceiver

BATTERY

HDET

INNER CHG

+2.9

V_Rx_LDO

DFM

Processor

CDMA

Processor

RF Controls

2

Speaker

LDO

CAMIF

Bluetooth

Baseband

Processor

B/T Chip

Antenna

Bluetooth RF

2


PAGE  
VX8600


Features of AX8600
1. Wave Type

( CELLULAR : G7W
    ( PCS : G7W
2. Frequency Scope
	Transmit Frequency (MHz)
	Receive Frequency (MHz)

	CELLULAR
	PCS
	CELLULAR
	PCS
	GPS

	824.82 ~ 848.19
	1850~1910
	869.82~893.19
	1930~1990
	1575.42


3. Rated Output Power :  CELLULAR = 0.20W 

                       PCS = 0.20W
4. Output Conversion Method : This is possible by correcting the key board channel. 

5. Voltage and Current Value of Termination Part Amplifier (Catalogue included)

	MODE
	Part Name
	Voltage
	Current
	Power

	CELLULAR 
	AWT6314R
	4.2V
	600mA
	0.20W

	PCS
	AWT6314R
	4.2V
	600mA
	0.20W


6. Functions of Major Semi-Conductors 

	Classification
	Function

	MSM6500
	Terminal operation control and digital signal processing 

	Memory MCP (HYG0UEG0AF1P-6SS0E)
	Flash Memory (512Mbit) + SDRAM (512Mbit)

 Storing of terminal operation program

	RFR6500
	Converts Rx RF signal to baseband signal

	RFT6150
	Converts baseband signal to Tx RF signal


7. Frequency Stability
    ( CELLULAR : ±0.5PPM 

( PCS : ±0.1PPM

[image: image129.wmf]Peripheral Circuits

PLL

HK ADC

Microprocessor Bus

Address/Data

General

-

Purpose Interface Bus

JTAG Interfaces

Rx

ADCs

RF

Interface

TX

DACs

AUX

SBI

SBI

Vocoder

EVRC

13

k

MP4

MIDI

QDSP4000

Integrated

CODEC

MODE Select

Interface

ETM

gpsOne

Processor

UART1

UART2

UART3/USB

ARM9

TDMI

Memory

Controller

General

-

Purpose

Interface

Camera module

(1.3M C

-

MOS)

PC

Connectivity

Test/Debug

System

Digital Test Bus

JTAG

Keypad input

RFT6100

RFR6000

MSM6500

Mobile Station Modem

VCTCXO

19.2

MHz

1

2

3

4

5

6

7

8

9

*

#

0

POWER

Amplifier

LCD

Memory

NAND Flash(512Mb)

LP

-

SDRAM(256Mb)

EAR

External

EAR/MIC

KEYPAD, BEZEL,

SIIDEKEY

MIC

Audio AMP

Data Comm

DM

USB

USB

Transceiver

BATTERY

HDET

INNER CHG

+2.9

V_Rx_LDO

DFM

Processor

CDMA

Processor

RF Controls

2

Speaker

LDO

CAMIF

Bluetooth

Baseband

Processor

B/T Chip

Antenna

Bluetooth RF

2

Peripheral Circuits

PLL

HK ADC

Microprocessor Bus

Address/Data

General

-

Purpose Interface Bus

JTAG Interfaces

Rx

ADCs

RF

Interface

TX

DACs

AUX

SBI

SBI

Vocoder

EVRC

13

k

MP4

MIDI

QDSP4000

Integrated

CODEC

MODE Select

Interface

ETM

gpsOne

Processor

UART1

UART2

UART3/USB

ARM9

TDMI

Memory

Controller

General

-

Purpose

Interface

Camera module

(1.3M C

-

MOS)

PC

Connectivity

Test/Debug

System

Digital Test Bus

JTAG

Keypad input

RFT6150

RFR6500

MSM6500

Mobile Station Modem

VCTCXO

19.2

MHz

1

2

3

4

5

6

7

8

9

*

#

0

POWER

Amplifier

LCD

Memory

NAND Flash(512Mb)

LP

-

SDRAM(512Mb)

EAR

External

EAR/MIC

KEYPAD, BEZEL,

SIIDEKEY

MIC

Audio AMP

Data Comm

DM

USB

USB

Transceiver

BATTERY

HDET

INNER CHG

+2.9

V_Rx_LDO

DFM

Processor

CDMA

Processor

RF Controls

2

Speaker

LDO

CAMIF

Bluetooth

Baseband

Processor

B/T Chip

Antenna

Bluetooth RF

2

            CDMA Mobile Subscriber Unit

     AX8600 

[image: image130.emf] 

The   Pr o b l e m   m a y  b e L o g i c   pa r t   R e f e r  t o  L o g i c  t r o u b l e s h oot    


DUAL BAND CDMA [PCS/Cellular/w/GPS]

 CDMA MOBILE PHONE
Table of Contents

General Introduction

         
 4

CHAPTER 1. System Introduction
  
31. System Introduction


32. Features and Advantages of CDMA Mobile Phone 


33. Structure and Functions of dual-band CDMA Mobile Phone


34. Specification


5. Installation
18

CHPATER 2. NAM Input Method

1. NAM Program Method and Telephone Number Inputting Method
20

CHAPTER 3. Circuit Description

1. RF Transmit/Receive Part
7
2. Digital/Voice Processing Part
32
CHAPTER 4. Trouble Shooting…………………………………………………………………….38
CHAPTER 5. Safety……………………………………………………………………………..…..44

CHAPTER 6. Glossary…………………………………………………………………………..…..47

APPENDIX……………………………………………………………………………………….…..60




[image: image131.emf] 

The   Pr o b l e m   m a y  b e L o g i c   pa r t   R e f e r  t o  L o g i c  t r o u b l e s h oot    


 The AX8600 phone has been designed to operate on the latest digital mobile communication technology, Code Division Multiple Access (CDMA). This CDMA digital technology has greatly enhanced voice clarity and can provide a variety of advanced features. Currently, CDMA mobile communication technology has been commercially used in Cellular and Personal Communication Service (PCS). The difference between them is the operating frequency spectrum. Cellular uses 800MHz and PCS uses 1.9GHz. The AX8600 support GPS Mode, we usually call it tri-band phone. Also, AX8600 works on Advanced Mobile Phone Service (S-GPS). We call it dual-mode phone. If one of the Cellular, PCS base stations is located nearby, Call fail rate of triple-mode phone is less than dual-mode phone or single-mode phone.

 The CDMA technology adopts DSSS (Direct Sequence Spread Spectrum). This feature of DSSS enables the phone to keep communication from being crossed and to use one frequency channel by multiple users in the same specific area, resulting that it increases the capacity 10 times more compared with that in the analog mode currently used. Soft/Softer Handoff, Hard Handoff, and Dynamic RF power Control technologies are combined into this phone to reduce the call being interrupted in a middle of talking over the phone.

Cellular and PCS CDMA network consists of MSO (Mobile Switching Office), BSC (Base Station Controller), BTS (Base station Transmission System), and MS (Mobile Station).  The following table lists some major CDMA Standards. 

	CDMA Standard
	Designator
	Description

	Basic air interface
	TIA/EIA/IS-95-A/B/C

ANSI J-STD-008
	Protocol between MS and BTS for Cellular & AMPS
Protocol between MS and BTS for PCS

	Network
	TIA/EIA/IS-634

TIA/EIA/IS/651

TIA/EIA/IS-41-C

TIA/EIA/IS-124
	MAS-BS

PCSC-RS

Intersystem operations

Nom-signaling data comm.

	Service
	TIA/EIA/IS-96-B

TIA/EIA/IS-99

TIA/EIA/IS-637

TIA/EIA/IS-657
	Speech CODEC

Assign data and fax

Short message service

Packet data

	Performance
	TIA/EIA/IS-97

TIA/EIA/IS-98

ANSI J-STD-018

ANSI J-STD-019

TIA/EIA/IS-125
	Cellular base station

Cellular mobile station

PCS personal station

PCS base station

Speech CODEC


* TSB –74: Protocol between an IS-95A system and ANSI J-STD-008
[image: image132.wmf]
1.1 CDMA Abstract

The CDMA mobile communication system has a channel hand-off function that is used for collecting the information on the locations and movements of mobile telephones from the cell site by automatically controlling several cell site through the setup of data transmission routes, and then enabling one switching system to carry out the automatic remote adjustment. This is to maintain continuously the call state through the automatic location confirmation and automatic radio channel conversion when the busy subscriber moves from the service area of one cell site to that of another by using automatic location confirmation and automatic radio channel conversion functions. The call state can be maintained continuously by the information exchange between switching systems when the busy subscriber moves from one Cellular system area to the other Cellular system area.
In the Cellular system, the cell site is a small-sized low output type and utilizes a frequency allocation system that considers mutual interference, in an effort to enable the re-use of corresponding frequency from a cell site separated more than a certain distance. 

Unlike the time division multiple access (TDMA) or frequency division multiple access (FDMA) used in the band limited environment, the Code Division Multiple Access (CDMA) system which is one of digital Cellular systems is a multi-access technology under the interference limited environment. It can process more number of subscribers compared to other systems (TDMA system has the processing capacity three times greater than the existing FDMA system whereas CDMA system, about 12~15 times of that of the existing system).
CDMA system can be explained as follows; TDMA or CDMA can be used to enable each person to talk alternately or provide a separate room for each person when two persons desire to talk with each other at the same time, whereas FDMA can be used to enable one person to talk in soprano, whereas the other in bass (one of the two talkers can carry out synchronization for hearing in case there is a bandpass filter function in the area of the hearer). Another available method is to make two persons to sing in different languages at the same time, space, and frequency when wishing to let the audience hear the singing without being confused. This is the characteristic of CDMA.
On the other hand, when employing the CDMA technology, each signal has a different pseudo-random binary sequence used to spread the spectrum of carrier. A great number of CDMA signals share the same frequency spectrum. In the perspective of frequency area or time area, several CDMA signals are overlapped. Among these types of signals, only desired signal energy is selected and received through the use of pre-determined binary sequence; desired signals can be separated, and then received with the correlator used for recovering the spectrum into its original state. At this time, the spectrums of other signals that have different codes are not recovered into its original state, and appears as the self-interference of the system.
2. Features and Advantages of CDMA Mobile Phone 
2.1 Various Types of Diversities

When employing the narrow band modulation (30kHz band) that is the same as the analog FM modulation system used in the existing Cellular system, the multi-paths of radio waves create a serious fading. However, in the CDMA broadband modulation (1.25MHz band), three types of diversities (time, frequency, and space) are used to reduce serious fading problems generated from radio channels in order to obtain high-quality calls.
Time diversity can be obtained through the use of code interleaving and error correction code whereas frequency diversity can be obtained by spreading signal energy to wider frequency band. The fading related to normal frequency can affect the normal 200~300KHz among signal bands and accordingly, serious effect can be avoided. Moreover, space diversity (also called path diversity) can be realized with the following three types of methods. 
First, it can be obtained by the duplication of cell site receive antenna. Second, it can be obtained through the use of multi-signal processing device that receives a transmit signal having each different transmission delay time and then, combines them. Third, it can be obtained through the multiple cell site connection (Soft Handoff) that connects the mobile station with more than two cell sites at the same time. 

2.2 Power Control

The CDMA system utilizes the forward (from a base station to mobile stations) and backward (from the mobile station to the base station) power control in order to increase the call processing capacity and obtain high-quality calls. In case the originating signals of mobile stations are received by the cell site in the minimum call quality level (signal to interference) through the use of transmit power control on all the mobile stations, the system capacity can be maximized.If the signal power of mobile station is received too strong, the performance of that mobile station is improved. However, because of this, the interference on other mobile stations using the same channel is increased and accordingly, the call quality of other subscribers is reduced unless the maximum accommodation capacity is reduced.
In the CDMA system, forward power control, backward open loop power control, and closed loop power control methods are used. The forward power control is carried out in the cell site to reduce the transmit power on mobile stations less affected by the multi-path fading and shadow phenomenon and the interference of other cell sites when the mobile station is not engaged in the call or is relatively nearer to the corresponding cell site. This is also used to provide additional power to mobile stations having high call error rates, located in bad reception areas or far away from the cell site.
The backward open loop power control is carried out in a corresponding mobile station; the mobile station measures power received from the cell site and then, reversely increases/decreases transmit power in order to compensate channel changes caused by the forward link path loss and terrain characteristics in relation to the mobile station in the cell site. By doing so, all the mobile transmit signals received by the base station have same strength.
 Moreover, the backward closed loop power control used by the mobile station is performed to control power using the commands issued out by the cell site. The cell site receives the signal of each corresponding mobile station and compares this with the pre-set threshold value and then, issues out power increase/decrease commands to the corresponding mobile station every 1.25msec (800 times per second). By doing so, the gain tolerance and the different radio propagation loss on the forward/backward link are complemented. 
2.3 Voice Encoder and Variable Data Speed 

 The bi-directional voice service having variable data speed provides voice communication which employs voice encoder algorithm having power variable data rate between the base station and the mobile station. On the other hand, the transmit voice encoder performs voice sampling and then, creates encoded voice packets to be sent out to the receive voice encoder, whereas the receive voice encoder demodulates the received voice packets into voice samples. 

 One of the two voice encoders described in the above is selected for use depending on inputted automatic conditions and message/data; both of them utilize four-stage frames of 9600, 4800, 2400, and 1200 bits per second for Cellular and 14400,7200,3600,1800 bits per second for PCS, so PCS provide relatively better voice quality (almost twice better than the existing celluar system). In addition, this type of variable voice encoder utilizes adaptive threshold values on selecting required data rate. It is adjusted in accordance with the size of background noise and the data rate is increased to high rate only when the voice of caller is inputted. 

 Therefore, background noise is suppressed and high-quality voice transmission is possible under the environment experiencing serious noise. In addition, in case the caller does not talk, data transmission rate is reduced so that the transmission is carried out in low energy. This will reduce the interference on other CDMA signals and as a result, improve system performance (capacity increased by about two times).
2.4 Protecting Call Confidentiality
Voice privercy is provided in the CDMA system by means of the private long code mask used for PN spreading. Voice privacy can ve applied on the traffic channels only. All calls are initiated using the public long code mask for PN spreading. The mobile station user may request voice privacy during call setup using the origination message or page response message, and during traffic channel operation using the long code transition request order.

The Transition to private long code mask will not be performed if authentication is not performed. To initiate a transition to the private or public long code mask, either the base station or the mobile station sends a long code transition request order on the traffic channel.

2.5 Soft Handoff

A handoff in which the mobile station commences communications with a new base station without interrupting communications with the old base station. Soft handoff can only be used between CDMA channels having identical freqeuncy assignments.

2.6 Frequency Re-Use and Sector Segmentation 

 Unlike the existing analog Cellular system, the CDMA system can reuse the same frequency at the adjacent cell. there is no need to prepare a separate frequency plan. Total interference generated on mobile station signals received from the cell site is the sum of interference generated from other mobile stations in the same cell site and interference generated from the mobile station of adjacent cell site. That is, each mobile station signal generates interference in relation to the signals of all the other mobile stations. 

 Total interference from all the adjacent cell sites is the ratio of interference from all the cell sites versus total interference from other mobile stations in the same cell site (about 65%). In the case of directional cell site, one cell normally uses a 120°sector antenna in order to divide the sector into three. In this case, each antenna is used only for 1/3 of mobile stations in the cell site and accordingly, interference is reduced by 1/3 on the average and the capacity that can be supported by the entire system is increased by three times. 

2.7 Soft Capacity

 The subscriber capacity of the CDMA system is flexible depending on the relation between the number of users and service classes. For example, the system operator can increase the number of channels available for use during the busy hour despite the drop in call quality. This type of function requires 40% of normal call channels in the standby mode during the handoff, in an effort to avoid call disconnection resulting from the lack of channels. 

In addition, in the CDMA system, services and service charges are classified further into different classes so that more transmit power can be allocated to high class service users for easier call set-up; they can also be given higher priority of using hand-off function than the general users. 
3. Structure and Functions of tri-band CDMA Mobile Phone 

 The hardware structure of CDMA mobile phone is made up of radio frequency (RF) part and logic part. The RF part is composed of Receiver part (Rx), Transmitter part (Tx) and Local part (LO). For the purpose of operating on tri-band, It is necessary dual Tx path, tri Rx path, dual PLL and switching system for band selection. The mobile phone antenna is connected with the frequency seperater which divide antenna input/output signals between Cellular frequency band (824~894 MHz) and PCS frequency band (1850~1990MHz). Each separated path is linked with the Cellular duplexer and PCS duplexer. Duplexer carrys out seperating Rx band and Tx band. The Rx signals from the antenna are converted into intermediate frequency(IF) band by the frequency synthesizer and frequency down converter. And then, pass SAW filter which is a band pass filter for removing out image frequency. The IF output signals that have been filtered is converted into digital signals via Analog-to-Digital Converter (ADC). In front of the ADC, switching system is required to choose which band path should be open. The digital signals send to 5 correlators in each CDMA de-modulator. Of these, one is called a searcher whereas the remaining 4 are called data receivers (fingers). Digitalized IF signals include a great number of call signals that have been sent out by the adjacent cells. These signals are detected with pseudo-noise sequence (PN Sequence). Signal to interference ratio (C/I) on signals that match the desired PN sequence are increased through this type of correlation detection process, but other signals obtain processing gain by not increasing the ratio. The carrier wave of pilot channel from the cell site most adjacently located is demodulated in order to obtain the sequence of encoded data symbols. During the operation with one cell site, the searcher searches out multi-paths in accordance with terrain and building reflections. On three data receivers, the most powerful 3 paths are allocated for the parallel tracing and receiving. Fading resistance can be improved a great deal by obtaining the diversity combined output for de-modulation. Moreover, the searcher can be used to determine the most powerful path from the cell sites even during the soft handoff between the two cell sites. Moreover, 3 data receivers are allocated in order to carry out the de-modulation of these paths. Output data that has been demodulated changes the data string in the combined data row as in the case of original signals(deinterleaving), and then, are demodulated by the forward error correction decoder which uses the Viterbi algorithm.

 Mobile station user information send out from the mobile station to the cell site pass through the digital voice encoder via a mike. Then, they are encoded and forward errors are corrected through the use of convolution encoder. Then, the order of code rows is changed in accordance with a certain regulation in order to remove any errors in the interleaver. Symbols made through the above process are spread after being loaded onto PN carrier waves. At this time, PN sequence is selected by each address designated in each call. 

Signals that have been code spread as above are digital modulated (QPSK) and then, power controlled at the automatic gain control amplifier (AGC Amp). Then, they are converted into RF band by the frequency synthesizer synchronizing these signals to proper output frequencies. 

Transmit signals obtained pass through the duplexer filter and then, are sent out to the cell site via the antenna. 
4. Specification
4.1 General Specification

4.1.1 Transmit/Receive Frequency Interval : 

1) CELLULAR : 45 MHz
2) PCS : 80 MHz
4.1.2 Number of Channels (Channel Bandwidth)

1) CELLULAR : 20 Channels

2)  PCS : 48 Channels

4.1.3 Operating Voltage : DC 3.3~4.2V

4.1.4 Battery Power Consumption : DC 3.7V

	
	SLEEP
	IDLE
	MAX POWER

	CELLULAR
	1 mA 
	110~140mA
	    700 mA (24 dBm)

	PCS
	1 mA
	120~150 mA
	    700 mA (24 dBm)


4.1.5 Operating Temperature : -0(C ~ +60(C

4.1.6 Frequency Stability
1) CDMA : ±0.5PPM

2) PCS : ±0.1PPM

4.1.7 Antenna : Stubby Type (Helical), 50 (
4.1.8 Size and Weight
1) Size : 98.7 * 47.8 * 17.4 mm
2) Weight : 103 g (Approximately with standard battery)
4.1.9 Channel Spacing

1) CELLULAR : 1.25MHz

2) PCS: 1.25 MHz

4.1.10 Battery Type, Capacity and Operating Time. 
                                                              Unit = Hours : Minutes 
	
	Standard (800mAh)

	Standby Time
	CELLULAR
	About 150 Hours (SCI=1, Rx Power –75dBm)

	
	PCS
	About 150 Hours (SCI=1, Rx Power –75dBm)

	Talk time
	CELLULAR
	200 Minutes (Rx Power –92dBm)

	
	PCS
	200 Minutes (Rx Power –92dBm)


4.2 Receive Specification 

4.2.1 Frequency Range
1) CELLULAR : 869.820 MHz ~ 893.190 MHz

2) PCS : 1930 MHz ~ 1990 MHz

3) GPS : 1575.42 MHz
4.2.2 Local Oscillating Frequency Range : 

1) CELLULAR : 1738.08MHz ~ 1787.94MHz

2) PCS : 1715.56MHz ( 1768.89MHz 

3) GPS : 3150.84MHz
4.2.3 Sensitivity
1) CELLULAR : -104dBm (C/N 12dB or more)

2) PCS : -104dBm (C/N 12dB or more)
3) GPS : -148.5dBm (without SA mode)

4.2.4 Selectivity  
1) CELLULAR : 3dB C/N Degration (With Fch±1.25 kHz : -30dBm)
2) PCS : 3dB C/N Degration (With Fch±1.25 kHz : -30dBm)
4.2.5 Spurious Wave Suppression : Maximum of -80dB

4.2.6 CDMA Input Signal Range
· Dynamic area of more than -104~ -25 dB: 79dB at the 1.23MHz band.
4.3 Transmit Specification 
4.3.1 Frequency Range
1) CELLULAR : 824.820MHz ~ 848.190MHz

2) PCS : 1850 MHz ~ 1910 MHz

4.3.2 Output Power

1) CELLULAR : 0.224W
2) PCS: 0.224W

4.3.3 Interference Rejection 

1) Single Tone : -30dBm at 900 kHz (CELLULAR), -30dBm at 1.25MHz(PCS)

2) Two Tone  : -43dBm at 900 kHz & 1700kHz(CELLULAR), -43dBm at 1.25 MHz & 2.05 MHz (PCS)

4.3.11 CDMA TX Frequency Deviation : 

1) CELLULAR: +300Hz or less

2) PCS: ( 150Hz 

4.3.12 CDMA TX Conducted Spurious Emissions 

1) CELLULAR :  900kHz : - 42 dBc/30kHz below

                 1.98MHz : - 54 dBc/30kHz below

2) P   C  S  :  1.25MHz : - 42 dBc/30kHz below

1.98MHz : - 50 dBc/30kHz below

4.3.13 CDMA Minimum TX Power Control

   1) CELLULAR : - 50dBm below 

   2) P    C   S : -50dBm below
4.4 MS (Mobile Station) Transmitter Frequency

4.4.1 CELLULAR mode

	Ch #
	Center Freq. (MHz)
	Ch #
	Center Freq. (MHz)

	1011

29

70

111

152

193

234

275

316

363
	824.640 

825.870 

827.100 

828.330 

829.560 

830.790 

832.020 

833.250 

834.480 

835.890 
	404

445

486

527

568

609

650

697

738

779
	837.120 

838.350 

839.580 

840.810 

842.040 

843.270 

844.500 

845.910 

847.140 

848.370 


4.4.2 PCS mode

	Ch #
	Center Freq (MHz)
	Ch #
	Center Freq (MHz)
	Ch #
	Center Freq (MHz)

	25
	1851.25 
	425
	1871.25 
	825
	1891.25 

	50
	1852.50 
	450
	1872.50 
	850
	1892.50 

	75
	1853.75 
	475
	1873.75 
	875
	1893.75 

	100
	1855.00 
	500
	1875.00 
	900
	1895.00 

	125
	1856.25 
	525
	1876.25 
	925
	1896.25 

	150
	1857.50 
	550
	1877.50 
	950
	1897.50 

	175
	1858.75 
	575
	1878.75 
	975
	1898.75 

	200
	1860.00 
	600
	1880.00 
	1000
	1900.00 

	225
	1861.25 
	625
	1881.25 
	1025
	1901.25 

	250
	1862.50 
	650
	1882.50 
	1050
	1902.50 

	275
	1863.75 
	675
	1883.75 
	1075
	1903.75 

	300
	1865.00 
	700
	1885.00 
	1100
	1905.00 

	325
	1866.25 
	725
	1886.25 
	1125
	1906.25 

	350
	1867.50 
	750
	1887.50 
	1150
	1907.50 

	375
	1868.75 
	775
	1888.75 
	1175
	1908.75 


4.5 MS (Mobile Station) Receiver Frequency

  4.5.1 CELLULAR mode

	Ch. #
	Center Freq. (MHz)
	Ch. #
	Center Freq. (MHz)

	1011

29

70

111

152

193

234

275

316

363
	869.640 

870.870 

872.100 

873.330 

874.560 

875.790 

877.020 

878.250 

879.480 

880.890 
	404

445

486

527

568

609

650

697

738

779
	882.120 

883.350 

884.580 

885.810 

887.040 

888.270 

889.500 

890.910 

892.140 

893.370 


4.5.2 PCS mode

	Ch #
	Center Freq (MHz)
	Ch #
	Center Freq (MHz)
	Ch #
	Center Freq (MHz)

	25
	1931.25 
	425
	1951.25 
	825
	1971.25 

	50
	1932.50 
	450
	1952.50 
	850
	1972.50 

	75
	1933.75 
	475
	1953.75 
	875
	1973.75 

	100
	1935.00 
	500
	1955.00 
	900
	1975.00 

	125
	1936.25 
	525
	1956.25 
	925
	1976.25 

	150
	1937.50 
	550
	1957.50 
	950
	1977.50 

	175
	1938.75 
	575
	1958.75 
	975
	1978.75 

	200
	1940.00 
	600
	1960.00 
	1000
	1980.00 

	225
	1941.25 
	625
	1961.25 
	1025
	1981.25 

	250
	1942.50 
	650
	1962.50 
	1050
	1982.50 

	275
	1943.75 
	675
	1963.75 
	1075
	1983.75 

	300
	1945.00 
	700
	1965.00 
	1100
	1985.00 

	325
	1946.25 
	725
	1966.25 
	1125
	1986.25 

	350
	1947.50 
	750
	1967.50 
	1150
	1987.50 

	375
	1948.75 
	775
	1968.75 
	1175
	1988.75 


4.5.3 GPS mode     : 1575.42 MHz

4.5.4 Bluetooth mode : 2400 MHz ~ 2483.5 MHz
4.6  AC Adaptor           : See Appendix

4.7  Cigar Lighter Charger  : See Appendix

4.7  Hands – Free Kit       : See Appendix
5. Installation

5.1 Installing a Battery Pack
1) The Battery pack is keyed so it can only fit one way. Align the groove in the battery pack with the rail on the back of the phone until the battery pack rests flush with the back of the phone.

2) Slide the battery pack forward until you hear a “click”, which locks the battery in place.

5.2 For Adapter Use

1) Plug the adapter into a wall outlet. The adapter can be operated from a 110V source. When AC power is connected to the adapter.

2) Insert the adapter IO plug into the phone with the installed battery pack. 

Red light indicates battery is being charged.. Green light indicates battry is fully charged.

5.3 For Mobile Mount

5.3.1 Installation Position

In order to reduce echo sound when using the Hands-Free Kit, make sure that the speaker and microphone are not facing each other and keep microphone a generous distance from the speaker.

5.3.2 Cradle Installation

Choose an appropriate flat surface where the unit will not interface with driver’s movement or passenger’s  comfort. The driver/user should be able to access the phone with ease. Using the four self-tapping screws provided, mount the supplied braket on the selected area. Then with the four machine screws provided, mount the counterpart on the reverse side of the reverse side of the cradle. Secure the two brackets firmly together by using the two bracket joint screws provide. The distance between the cradle and the interface box must not exceed the length of the main cable.

5.3.3 Interface Box
Choose an appropriate flat surface ( somewhere under the dash on the passenger side is preferred ) and mount the IB bracket with the four self-tapping screws provided. Clip the IB into the IB bracket.

5.3.4. Microphone Installation
Install the microphone either by cliiping I onto the sunvisor (driver’s side) or by attaching it to door post (driver’s side), using a velcno adhesive tape (not included).  

5.3.5 Cable Connections

5.3.5.1 Power and Ignition Cables

Connect the red wire to the car battery positive terminal and the black wire to the car ground. Connect the green wire to the car ignition sensor terminal. ( In order to operate HFK please make sure to connect green wire to ignition sensor terminal.) Connect the kit’s power cable connector to the interface box power receptacle.

5.3.5.2 Antenna Cable Connection

Connect the antenna coupler cable connector from the cradle to the internal antenna connector. ( Antenna is not included.)

[image: image133.wmf] 


1) NAM Programming Method 

1. Press ‘menu’ + 0 + “000000”

[image: image1.png]


 
[image: image2.png]



2.   Press ‘1’ key for entering ‘Service Prg.’

[image: image3.png]



● Usually pressing ‘OK’ will save the change.

● To exit Service Program, press ‘END’ key.

2-1) ESN
You can see the ESN number.

Press ‘OK Key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to exit Service Programming’.


[image: image4.png]



2-2) NAM1 Phone Number (MDN)
You can edit NAM1 Phone Number(MDN).

Press OK key to edit more NAM1 items.

Press soft key 2 to edit previous NAM1 items.


[image: image5.png]9196143309




2-3) NAM1 Phone Number (MIN)
You can edit NAM1 Phone Number(MIN).

Press OK key to edit more NAM1 items.

Press soft key 2 to edit previous NAM1 items.


[image: image6.png]9196143309




2-4) NAM1 Home SID
You can edit NAM1 Home SID.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image7.png]



2-4) NAM1 Name
You can edit NAM1 Name.

Press ‘OK’ to edit more NAM1 items.

Press ‘Soft Key 1’ to edit NAM1 Name.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image8.png]NAM1




2-5) More NAM1 Programming
You can decide to edit more NAM1 items.

Press ‘OK key’ to exit Service Programming.

Press ‘Soft Key 2’ to edit more advanced NAM1 items.


[image: image9.png]Basic NAM1 is completed





2-6) Service Code
You can edit Service Code.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image10.png]



2-7) NAM1 MCC
You can edit NAM1 Mobile Country Code.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image11.png]311





2-8) NAM1 NMSID
You can edit NAM1 NMSID.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image12.png]279196143309




2-9) NAM1 True IMSI MCC
You can edit NAM1 True IMSI MCC.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image13.png]311





2-10) NAM1 True IMSI NMSID
You can edit NAM1 True IMSI NMSID.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image14.png]270000000172




2-11) NAM1 PRL Enabled
You can see NAM1 PRL Enabled.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image15.png]



2-12) CDMA Home SID/NID
You can edit NAM1 Home SID/NID Pairs.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image16.png]



2-13) NAM1 CDMA Pri. CH A
You can edit NAM1 CDMA Primary Channel A.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image17.png]



2-14) NAM1 CDMA Sec. CH A
You can edit NAM1 CDMA Secondary Channel A.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image18.png]691




2-15) NAM1 CDMA Pri. CH B
You can edit NAM1 CDMA Primary Channel B.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image19.png]



2-16) NAM1 CDMA Sec. CH B
You can edit NAM1 CDMA Secondary Channel B.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image20.png]



2-17) Lockout SID/NID
You can edit Lockout SID/NID Pairs.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.


[image: image21.png]



2-18) NAM1 Home Sys Reg
You can edit Home System Registration.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.

Press Left, Right, Up, Down key to toggle Yes/No.


[image: image22.png]



2-19) NAM1 Forn SID Reg
You can edit NAM1 Foreign SID Registration.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.

Press Left, Right, Up, Down key to toggle Yes/No.


[image: image23.png]



2-20) NAM1 Forn NID Reg
You can edit NAM1 Foreign NID Registration.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.

Press Left, Right, Up, Down key to toggle Yes/No.


[image: image24.png]



2-21) NAM1 Acc Ovld Class
You can see NAM1 Access Overload Class.

Press ‘OK key’ to edit more NAM1 items.

Press ‘Soft Key 2’ to edit previous NAM1 items.

[image: image25.png]




2-22) NAM2 Setting
You can decide to edit NAM2 items.

Press ‘OK key’ to skip NAM2 items settings.

Press ‘Soft Key 2’ to edit NAM2 related items.


[image: image26.png]NAM? Setting





2-23) Phone Model
You can see the Phone Model number.

Press ‘OK key’ to edit more items.

Press ‘Soft Key 2’ to edit previous items.


[image: image27.png]



2-24) Slot Cycle Index
You can edit Slot Cycle Index

Press ‘OK key’ to save Slot Cycle Index.

Press ‘Soft Key 2’ to edit previous items.


[image: image28.png]



2-25) Powering Down 

Restart.


[image: image29.png]



[image: image134.wmf] 


1. RF Transmit/Receive Part 

1.1 Overview 

The TX and RX part employs the Direct-Conversion system. The TX and RX frequencies are respectively 824.04~848.97 and 869.04~893.97 for cellular and 1850~1910 and 1930~1990 for PCS. The block diagram is shown in [Figure 1-1]. RF signals received through the antenna are seperated by the Triplexer. 

RF Signal fed into the low noise amplifier in RFR6500(LNA) through the duplexer. Then, they are fed into Mixer in RFR6500. In RFR6500, the RF signal is changed into baseband signal directly. Then, this signal is changed into digital signal by the analog to digital converter (ADC, A/D Converter), and the digital circuit part of the MSM(Mobile Station Modem) 6500 processes the data from ADC. The digital processing part is a demodulator. 
In the case of transmission, RFT6150 receives OQPSK-modulated anlaog signal from the MSM6500. 
The RFT6150 connects directly with MSM6500 using an analog baseband interface. In RFT6150, the baseband quadrature signals are upconverted to the Cellular or PCS frequency bands and amplified to provide signal drive capability to the power amp. 

After that, the RF signal is amplified by the Power Amp in order to have enough power for radiation. Finally, the RF signal is sent out to the cell site via the antenna after going through the duplexer.
[Figure 1-1] Block Diagram of AX8600
[image: image135.wmf][image: image136.jpg]


[image: image137.wmf]Peripheral Circuits

PLL

HK ADC

Microprocessor Bus

Address/Data

General

-

Purpose Interface Bus

JTAG Interfaces

Rx

ADCs

RF

Interface

TX

DACs

AUX

SBI

SBI

Vocoder

EVRC

13

k

MP4

MIDI

QDSP4000

Integrated

CODEC

MODE Select

Interface

ETM

gpsOne

Processor

UART1

UART2

UART3/USB

ARM9

TDMI

Memory

Controller

General

-

Purpose

Interface

Camera module

(1.3M C

-

MOS)

PC

Connectivity

Test/Debug

System

Digital Test Bus

JTAG

Keypad input

RFT6100

RFR6000

MSM6500

Mobile Station Modem

VCTCXO

19.2

MHz

1

2

3

4

5

6

7

8

9

*

#

0

POWER

Amplifier

LCD

Memory

NAND Flash(512Mb)

LP

-

SDRAM(256Mb)

EAR

External

EAR/MIC

KEYPAD, BEZEL,

SIIDEKEY

MIC

Audio AMP

Data Comm

DM

USB

USB

Transceiver

BATTERY

HDET

INNER CHG

+2.9

V_Rx_LDO

DFM

Processor

CDMA

Processor

RF Controls

2

Speaker

LDO

CAMIF

Bluetooth

Baseband

Processor

B/T Chip

Antenna

Bluetooth RF

2

Peripheral Circuits

PLL

HK ADC

Microprocessor Bus

Address/Data

General

-

Purpose Interface Bus

JTAG Interfaces

Rx

ADCs

RF

Interface

TX

DACs

AUX

SBI

SBI

Vocoder

EVRC

13

k

MP4

MIDI

QDSP4000

Integrated

CODEC

MODE Select

Interface

ETM

gpsOne

Processor

UART1

UART2

UART3/USB

ARM9

TDMI

Memory

Controller

General

-

Purpose

Interface

Camera module

(1.3M C

-

MOS)

PC

Connectivity

Test/Debug

System

Digital Test Bus

JTAG

Keypad input

RFT6150

RFR6500

MSM6500

Mobile Station Modem

VCTCXO

19.2

MHz

1

2

3

4

5

6

7

8

9

*

#

0

POWER

Amplifier

LCD

Memory

NAND Flash(512Mb)

LP

-

SDRAM(512Mb)

EAR

External

EAR/MIC

KEYPAD, BEZEL,

SIIDEKEY

MIC

Audio AMP

Data Comm

DM

USB

USB

Transceiver

BATTERY

HDET

INNER CHG

+2.9

V_Rx_LDO

DFM

Processor

CDMA

Processor

RF Controls

2

Speaker

LDO

CAMIF

Bluetooth

Baseband

Processor

B/T Chip

Antenna

Bluetooth RF

2


[image: image138.wmf] 


[image: image139.wmf] 


[image: image140.png]



[image: image141.png]



[image: image142.png]



[image: image143.png]



[image: image144.png]



[image: image145.png]



[image: image146.png]



[image: image147.png]



1.2 Description of Receive Part Circuit
1.2.1 Tri-plexer (U101)

The main function of Tri-plexer is to separating Cellular, PCS & GPS Signal in Tri-band system.

The detailed Specification of Tri-plexer is described below:

	
	
	Cellular
	PCS
	GPS
	Unit

	Insertion Loss
	1.25
	0.75
	1.5
	dB

	Attenuation
	Cellular
	
	9
	45
	dB

	
	PCS
	25
	
	48
	dB

	
	GPS
	20
	18
	
	dB

	Isolation 
	Cellular to ~
	
	28
	45
	dB

	
	PCS to ~
	10
	
	45
	dB

	
	GPS to ~
	45
	45
	
	dB

	Source Impedance
	50
	50
	50
	Ω

	Load Impedance
	50
	50
	50
	Ω


1.2.2 Duplexers (DP100, DP101)
The duplexer consists of the Rx bandpass filter (BPF) and the Tx BPF which has the function of separating Tx and Rx signals in the full duplex system for using the common antenna. The Tx part BPF is used to suppress noises and spurious out of the Tx frequency band. The Rx BPF is used to receive only Rx signal coming from the antenna, which is usually called preselector. It’s main function is to limit the bandwidth of spectrum reaching the LNA and mixer, attenuate receiver spurious response and suppress local oscillator energy. As a result frequency sensitivity and selectivity of mobile phone increase. The specification of AX8600 duplexer described below ;

· PCS duplexer
	
	Tx
	Rx
	Tx to Rx (min)

	Pass Band
	1850~1910 MHz
	1930~1990 MHz
	

	Insertion Loss
	3.5 dB max
	3.8 dB max
	

	Return Loss
	9.5 dB min
	9.5dB min
	

	Attenuation
	43 dB min (1930~1990MHz)
	52dB min (1850~1910MHz)
	54 dB (1850~1910MHz)

45 dB (1930~1990MHz)


· Cellular duplexer

	
	Tx
	Rx
	Tx to Rx (min)

	Pass Band
	824~849MHz
	869~894 MHz
	

	Insertion Loss
	2.5 dB max
	3.5 dB max
	

	Return Loss
	9dB min
	8dB min
	

	Attenuation
	46 dB min (869~894MHz)
	53 dB min (824~849MHz)
	54 dB (824~849MHz)

45 dB (869~894MHz)


1.2.3  LNAs (U109)

The RFR6500 has cellular, PCS LNAs, respectively.  The characteristics of Low Noise Amplifier (LNA) are low noise figure, high gain, high intercept point and high reverse isolation. The frequency selectivity characteristic of mobile phone is mostly determined by LNA.

The specification of AX8600 LNAs are described below: 

· Cellular 

	
	High current mode
	Low current mode
	Passive status

	Power Gain

*Gain mode G0(G2)

*Gain mode G1(G3)
	 16dB

 4.0dB
	 14dB

  3.0dB
	-6.0dB

-20.0dB

	Noise Figure

Gain mode G0(G2)

 Gainmode G1(G3)
	1.4dB

    4.0dB
	1.4dB

   4.0dB
	7.0dB

20dB

	Input IP3

Gain mode G0(G2)

Gain mode G1(G3)
	8dB

5dB
	-5dB 

-5dB
	12dB

10dB


*Gain mode G0 and G1 operate in high and low current mode. 

*Gain mode G2 and G3 operate in Passive status.

· PCS

	
	High current mode
	Low current mode
	Passive status

	Power Gain

*Gain mode G0(G1)

Gain mode G2
	 16dB

X
	14dB

X
	-3.0dB

-20.0dB

	Noise Figure

Gain mode G0(G1)

  Gain mode G2
	1.6dB

      X
	1.6dB

X
	4.0dB

20dB

	Input IP3

Gain mode G0(G1)

Gain mode G2
	6dB

X
	-2dB 

X
	10dB

10dB


*Gain mode G0 operate in high and low current mode.

*Gain mode G1 and G2 operate in passive status.

1.2.4  Down-converter Mixers(RFR6500 : U109) 

The RFR6500 device performs signal down-conversion for Cellular, PCS and GPS tri-band applications.  It contains all the circuitry (with the exception of external filters) needed to support conversion of received RF signals to baseband signals. It consists of the three downconverting Mixers (Cellular, PCS and GPS), and an RX VCO and RX PLL circuit..

 The GPS LNA & mixers offer the most advanced and integrated CDMA Rx solution designed to meet cascaded Noise Figure (NF) and Third-order Intercept Point (IIP3) requirements of IS-98C and J-STD-018 specifications for Sensitivity, Two-Tone Intermodulation, and Single-tone Desense.

Operation modes and band selection are specially controlled from the Mobile Station Modem MSM6500.

 The specifications of AX8600 Mixers are described below:

· Cellular

	
	High current mode
	Mid current mode
	Low current mode

	Noise Figure

High Gain mode

Low Gain mode 
	 10dB

 25dB
	 10dB

  25dB
	9.0dB

25.0dB

	Input IP3

High Gain mode.

Low Gain mode.
	4.0dB

    -5.0dB
	4.0dB

   -5.0dB
	-10.0dB

-5.0dB

	Input IP2

High Gain mode.

Low Gain mode.
	56dB

30dB
	56dB 

30dB
	40dB

30dB


· PCS

	
	High current mode
	Mid current mode
	Low current mode

	Noise Figure

High Gain mode

Low Gain mode 
	 10dB

 25dB
	 10dB

  25dB
	9.0dB

25.0dB

	Input IP3

High Gain mode.

Low Gain mode.
	4.0dB

    -11.0dB
	-8.0dB

   -11.0dB
	-8.0dB

-11.0dB

	Input IP2

High Gain mode.

Low Gain mode.
	56dB

30dB
	56dB 

30dB
	40dB

30dB


1.2.5 GPS Pre-LAN(U102)

The characteristics of Low Noise Amplifier (LNA) are low noise figure, high gain, high intercept point and high reverse isolation. The frequency selectivity characteristic of mobile phone is mostly determined by LNA.The specification of AX8600 GPS LNA is described below

	Parameter
	GPS Band
	Units

	Gain
	17.7
	dB

	Noise Figure
	0.6
	dB

	1dB compression point
	14.4
	dBm

	IIP3
	6.5
	dBm


1.2.6 Rx RF SAW FILTER(F101)

The main function of Rx RF SAW filter is to attenuate mobile phone spurious frequency, attenuate noise amplified by the LNA and suppress second harmonic originating in the LNA. 

1.2.6 GPS RF SAW FILTER(F100)

The main function of GPS RF SAW filter is to attenuate mobile phone spurious frequency, attenuate noise amplified by the GPS Pre-LNA and suppress second harmonic originating in the GPS Pre-LNA.
1.3 Description of Transmit Part Circuit

1.3.1 RFT6150 (U110)

The RFT6150 baseband-to-RF Transmit Processor performs all Tx signal-processing functions required between digital baseband and the Power Amplifier Modulator (PAM). The baseband quadrature signals are upconverted to the Cellular or PCS frequency bands and amplified to provide signal drive capability to the PAM. The RFT6150 includes an mixers for up-converting analog baseband to RF, a programmable PLL for generating Tx LO frequency, cellular and PCS driver amplifiers and Tx power control through an 85 dB VGA. As added benefit, the single sideband upconversion eliminates the need for a band pass filter normally required between the upconverter and driver amplifier. 

I, I/, Q and Q/ signals proceed from the MSM6500 to RFT6150 are analog signal. In CDMA mode, These signals are modulated by Offset Quadrature Phase Shift King (OQPSK). I and Q are 90 deg. out of phase, and I and I/ are 180 deg.  The mixers in RFT6150 converts baseband signals into RF signals. After passing through the upconverters, RF signal is inputted into the Power AMP.

· RFT6150 Cellular and PCS CDMA RF Specifications

	Parame
	Condition
	Min.
	Typ.
	Max.
	Units

	Rated Output Power
	Average CDMA Cellular

Average CDMA PCS
	
	7

9
	
	dBm

dBm

	Min Output Power
	Average CDMA Cellular

Average CDMA PCS
	
	-75

-75
	
	dBm

dBm

	Rx band noise power
	CDMA Cellular

CDMA PCS
	
	-136

-133
	
	dBm/Hz

	ACPR
	Cellular: Fc(885kHz

PCS: Fc(1.25MHz
	
	-56

-57
	
	dBc/

30kHz


1.3.2 Power Amplifier( U107) 
The power amplifier that can be used in the PCS and CDMA mode has linear amplification capability and high efficiency. For higher efficiency, it is made up of one MMIC (Monolithic Microwave Integrated Circuit) for which RF input terminal and internal interface circuit are integrated onto one IC after going through the AlGaAs/GaAs HBT (heterojunction bipolar transistor) process. The module of power amplifier is made up of an output end interface circuit including this MMIC. The maximum power that can be inputted through the input terminal is +17dBm and conversion gain is about 28dB. RF transmit signals that have been amplified through the power amplifier are sent to the duplexer.

1.4 Description of Frequency Synthesizer Circuit 

1.4.1 Voltage Control Temperature Compensation Crystal Oscillator (VCTCXO, X101)
The temperature variation of mobile phone can be compensated by VCTCXO. The reference frequency of a mobile phone is 19.2 MHz.  The receiver frequency tuning signals called TRK_LO_ADJ from MSM as 0.5 V~2.5 V DC via R and C filter in order to generate the reference frequency of 19.2 MHz and input it into the frequency synthesizer.  Frequency stability depending on temperature is ±2.0 ppm.
2. Digital/Voice Processing Part
2.1 Overview 
The digital/voice processing part processes the user's commands and processes all the digital and voice signal processing in order to operate in the phone. The digital/voice processing part is made up of a keypad/LCD, receptacle part, voice processing part, mobile station modem part, memory part, and power supply part.
2.2 Configuration 
2.2.1 Keypad/LCD and Receptacle Part 

This is used to transmit keypad signals to MSM6500. It is made up of a keypad backlight part that illuminates the keypad, LCD part that displays the operation status onto the screen, and a receptacle that receives and sends out voice and data with external sources.
2.2.2 Voice Processing Part

The voice processing part is made up of an audio codec used to convert MIC signals into digital voice signals and digital voice signals into analog voice signals, amplifying part for amplifying the voice signals and sending them to the ear piece, amplifying part that amplifies ringer signals coming out from MSM6500, and amplifying part that amplifies signals coming out from MIC and transferring them to the audio processor. 

2.2.3 MSM (Mobile Station Modem) 6500 Part 

MSM is the core elements of CDMA terminal and carries out the functions of CPU, encoder, interleaver, deinterleaver, Viterbi decoder, Mod/Demod, and vocoder.

2.2.4 Memory Part
The memory part is made up of a NAND Flash memory and a SDRAM for storing data. 

2.2.5 Power Supply Part 
The power supply part is made up of circuits for generating various types of power, used for the digital/voice processing part. 
2.3 Circuit Description

2.3.1 Keypad/LCD and Receptacle Part

Once the keypad is pressed, the key signals are sent out to MSM6500 for processing. In addition, when the key is pressed, the keypad/LCD lights up through the use of  EL Sheet. The terminal status and operation are displayed on the screen for the user with the characters and icons on the LCD.
Moreover, it exchanges audio signals and data with external sources through the receptacle, and then receives power from the battery or external batteries.
2.3.2 Audio Processing Part 

MIC signals are amplified through OP AMP, inputted into the audio codec(included in MSM6500) and converted into digital signals. Oppositely, digital audio signals are converted into analog signals after going through the audio codec. These signals are amplified at the audio amplifier and transmitted to the ear-piece. The signals from MSM6500 activate the ringer by using signals generated in the timer in MSM6500.
2.3.3 MSM Part 

MSM6500 is the core element of CDMA system terminal that includes ARM926EJ-S microprocessor core. It supports both CDMA and Digital FM, operating in both the cellular and PCS spectrums. The subsystems within the MSM6500 include a CDMA processor, a DFM processor, a multi-standard Vocoder, an integrated CODEC with earpiece and microphone amplifiers, general-purpose ADC for subsytem monitoring, an ARM926EJ-S microprocessor, and an RS-232 serial interfaces supporting forward and reverse link MDR data communications of 230.4 Kbps simultaneously. And it also contains complete digital modulation and demodulation systems for both CDMA and AMPS cellular standards, as specified in IS-95-A/B/C.

In MSM, coded symbols are interleaved in order to cope with multi-path fading. Each data channel is scrambled by the long code PN sequence of the user in order to ensure the confidentiality of calls. Moreover, binary quadrature codes are used based on walsh functions in order to discern each channel. Data created thus are 4-phase modulated by one pair of Pilot PN code and they are used to create I and Q data. 
When received, I and Q data are demodulated into symbols by the demodulator, and then de-interleaved in reverse to the case of transmission. Then, the errors of data received from viterbi decoder are detected and corrected. They are voice-decoded at the vocoder in order to output digital voice data.
[image: image148.png]i




[Figure 2-2] Block Diagram of Digital/Voice Processing Part
2.3.4 Memory Part 

MCP contents 512Mbits NAND FLASH memory and 512Mbits SDRAM. In the NAND Flash Memory part of MCP are programs used for terminal operation. The programs can be changed through downloading after the assembling of terminals. On the SDRAM data generated during the terminal operation are stored temporarily. 
2.3.5 Power Supply Part 

When the battery rated voltage (+3.7V) is fed and the PWR key of keypad is pressed, the power-up circuitry in Power on & off part is activated by the PWR_ON_SW signal, and then the LDO regulators for MSM are operated and +1.4V_MSMC, +1.8V_MSMP1,+2.6V_MSMP2, and +2.6V_MSMA are generated.

The Rx part regulator (+2.9V_RX1/+2.9V_RX0) is operated by the control signal of SLEEP/ from MSM6500

The Tx part regulator (+2.9V_TX) is operated by the control signal of IDLE/ from MSM6500.
The Camera part regulators which is named Charge Pump [U600](+1.8V_CAMERA,+2.8V_CAMERA) are operated by the control signal of CAMERA_EN/from MSM6500
2.3.6 Logic Part

The logic part consists of internal CPU of MSM, RAM, MCP. The MSM6500 receives TCXO (=19.2MHz) from the X101 and controls the phone in CDMA modes.  The major components are as follows:

·  CPU

The ARM926J-S microprocessor includes a 3 stage pipelined RISC architecture, both 32-bit ARM and 16-bit THUMB instruction sets, a 32-bit address bus, and a 32-bit internal data bus. It has a high performance and low power consumption.

· MCP 
NAND Flash is used to store the terminal’s program. Using the down-loading program, the program can be changed even after the terminal is fully assembled.

SDRAM is used to store the internal flag information, call processing data, and timer data.

· KEYPAD

For key recognition, key matrix is setup using KYPD[1][3][5][7][9][11][13][15][17][19][21] signal from MSM6500. EL backlight circuitry are included in the keypad for easy operation in the dark.

· LCD MODULE

LCD module contains a controller which will display the information onto the LCD by 16-bit data from the MSM6500. It is also supplied stable 2.8V_LCD from U201 for fine view angle and LCD reflects to improve the display efficiency. 4 LEDs are used to display LCD backlight.

· . TOUCH MODULE

Touch module which will play V CAST music . when the flip is closed press any side key to unlock the touch sensitive keys, Touch module which are touch sensor IC and touch led is loacted in FPCB (U103,LED 100,LED 101,LED 102)
[image: image149.png]



[image: image150.png]



[image: image151.png]



[image: image152.png]



[image: image153.png]


[image: image154.png]


[image: image155.png]


[image: image156.png]


[image: image157.png]


[image: image158.png]


[image: image159.png]


[image: image160.png]


[image: image161.png]


[image: image162.png]


[image: image163.png]


[image: image164.png]


[image: image30.jpg]232ex% 10 L}

I




                  [image: image31.jpg]



[image: image165.png]



[image: image166.png]



[image: image167.png]



[image: image168.png]



[image: image169.png]—<|—i§?<

ocer

]

A

[y

P

| X oo —1 it
,_q_& Quasesture
<%

oo

T_l g

Tncr] [0 cunsin] =
i R e
T
RFT6150 veo

N
uerexs

!
o a1

%

x

o

ReESERVED:

— =

e g

:

o

RFR6500

0

b

“Single receiver MSM device




[image: image170.png]17:41:08 APR 09, 2005
KR 881.59 HHz

REr -25.0 aen T 10 ae 3814 dem  cieer
PEAK URTTE &
Los
It
e
@ noi0"a
REFUEUED
St view &
BLaNK &
na s
sgrg Trace
Eorr 133
fore
10878
START B7E.57 iRz STop g8E 52

RES BN 100 kHz vBH 30 kHz SuP 20.0 miec R




[image: image171.png]IM00us A CH S 204 e
B2

off





[image: image172.png]TCXO-EN > & L <+HVPHR

P +2. Bv_TEXD




[image: image173.png]17:3!

Rer -
PEAK
LoG
I
EE

na sel
st Fel
Core|

sTART

5:24 APR @3, 2085
25.0 d8n AT 10 a8

KR 881.59 HHz
23678 Ge

REFLEVEL
355 abn

576,52 iRz
RES BN 100 kHz

vBH 30 kHz

5707 86 52 Az
SuP 206 meeo

cLear
URTTE A

nax
noLn"a

viEw &

BLANK A

Trace
88

Hore
1ai's

®




[image: image174.png]1714,

Rer -
PEAK
LoG
I
EE

na sel
st fe
Core|

sTART

2135 APR @9, 2005
25.0 d8n AT 10 a8

KR 881.59 HHz
235153 Gn

REFLEVEL
355 abn

576,52 iRz
RES BN 100 kHz

vBH 30 kHz

5707 86 52 Az
SuP 206 meeo

cLear
URTTE A

nax
noLn"a

viEw &

BLANK A

Trace
88

Hore
1ai's

®



[image: image175.png]1731

Rer -
PEAK
LoG
I
EE

na sel
st fe
Core|

cente

8151 APR @9, 2005
25.0 d8n AT 10 a8

KR 1.9600% Hz
238785 GEn

REFLEVEL
355 abn

R 1.36000 6nz
Res B 100 Kz vBH 30 kHz

SFan 1050 iz
Tur' %076 meeo

cLear
URTTE A

nax
noLn"a

viEw &

BLANK A

Trace
88

Hore
1ai's

®



[image: image176.png]17331

Rer -
PEAK
LoG
I
EE

na sel
st fe
Core|

cente

0:35 APR @9, 2005

25.0 d8n AT 10 a8

KR 1.9600% GHz
238785 GEn

REFLEVEL
355 abn

R 1.36000 6nz
Res B 100 Kz

vBH 30 kHz

SFan 1050 iz
Tur' %076 meeo

cLear
URTTE A

nax
noLn"a

viEw &

BLANK A

Trace
88

Hore
1ai's

®



[image: image177.png]1733,

Rer -
PEAK
LoG
I
EE

na sel
st fe
Core|

cente

2:03 APR @9, 2005
25.0 d8n AT 10 a8

KR 1.9600% Hz
235707 GEn

REFLEVEL
355 abn

R 1.36000 6nz
Res B 100 Kz vBH 30 kHz

SFan 1050 iz
Tur' %076 meeo

cLear
URTTE A

nax
noLn"a

viEw &

BLANK A

Trace
88

Hore
1ai's

®



[image: image178.jpg]1Cell Li-on

Travel Adapter.
V@ 1A

BATT = REFzP

UVLO |p REF
on Linear
Charger Control THERMAL
SHUTDOWN



[image: image32.jpg]





[image: image33]

[image: image34.png]BT_EN D

AX_ENC> < V_CHAR
Reos
+HVPWA > o < USB_0E/
I < USB_SUSPEND
i
e
oo oo o Legs
Bl FEEEELEERLE H20meonamm e
BaTHe . Lz 5
%?%55%%?@2 e —y 1. 4v_memMe
neos beeag et
s
PS_HOLDC> + L PvEHOLD E B
ON-SW-SENSE/ < oo/ ™ e + —< VPR
PHA_ON_SWE>- £z . 2 preon e )
+3. OV_TFLASH PR Py o Povo B2 | s 2, s
PMIC_SCLC> oK MAX1B2EETLAT oM ~—<JUsSB_0—
PMIC_SDAC> 825 o ) oaTa vrem 008
+2. B5Y-BT; . 2} our7 o —= 24y <quss_0+
+VPWA > o 5| mg vaus 22 u{XXs <] +5v_US8
+3. OV_MOTOR: O ERE 1 2| oute ouTe a2t +2. BY_MSMP2
+2. 8V_RX0<T~ outs - Fov 2L USB_AX_DATA
e I N gl & 2 8 o2 T +2. 6V_MSMA
12.8v_Axt ° IHEITE ala il 8y Yanl, YgmeonT [ -
A 205 IR FEERE ER AL =Rl e |+ . | i
ancaw By EEEEEEEEEE Al 1o mI b I
TCSCHLALTEMIAR o g 143 T A LA - <JusE_vMO
noi2 o “ d + <7 USB_VFO
&7 ERE El t < +vPwA
NS 1008 R B 1. 8Y_MEMP1
+2. Bv-LCO
AC_OK/* o
v ; 7 ro +2. BY_TX
¢ z (8 HE
H e 2 | g
e 8 g
glal lo Is 81|y
s N g
8T8 TR

oy
5
E

¥

RESIN/
BAT_ADC






[image: image35.jpg]@GND PS_HOLD

RESIN MSMP TRST TMS
TDO RCLK TCK

VX8600
86V7V02-08

1

S

PEETTTITCCCCTLTTTTITCES
T

L )






 SHAPE  \* MERGEFORMAT 




[image: image36.png]2. BV._TCXD C>

ey

TRE_LO_ADJ

Liaa
L)
v @ -1
O e
gm0 Losee
s S R v
1 e = 1”1
I~ i ™ s
il NI .
cif8 el o
l 5L
_—
e
RO P P

TEXO

Lo
> MsM_

BT







`[image: image37.jpg]












[image: image38.png]2
i

<a+2.9v-AX0

I Fe

<+2. BV MEMPE

T om AN o
w4 1=
Eégﬁf’sfsﬁ’af Foso_Tn
= . e
]

FXQ_1.)
X0 1
FX0_B_t
RXD_G.F
AXA_T.
AX1T
X1,
AX10.

=

=

bbbl

Gvoo_TUNE

< AFR_FESERVE

PART






[image: image39.jpg]



[image: image40.jpg]232ex% 10 L}

I













 SHAPE  \* MERGEFORMAT 

[image: image41]




[image: image42.png]*in AHTWORK
this LNA out biss plock should be
located to the pin Zlna_olt







[image: image43.jpg]








[image: image44.png]






[image: image45.jpg]232ex% 10 L}

I

















[image: image46.jpg]





[image: image47.png]BT_EN D

AX_ENC> < V_CHAR
Reos
+HVPWA > o < USB_0E/
I < USB_SUSPEND
i
e
oo oo o Legs
Bl FEEEELEERLE H20meonamm e
BaTHe . Lz 5
%?%55%%?@2 e —y 1. 4v_memMe
neos beeag et
s
PS_HOLDC> + L PvEHOLD E B
ON-SW-SENSE/ < oo/ ™ e + —< VPR
PHA_ON_SWE>- £z . 2 preon e )
+3. OV_TFLASH PR Py o Povo B2 | s 2, s
PMIC_SCLC> oK MAX1B2EETLAT oM ~—<JUsSB_0—
PMIC_SDAC> 825 o ) oaTa vrem 008
+2. B5Y-BT; . 2} our7 o —= 24y <quss_0+
+VPWA > o 5| mg vaus 22 u{XXs <] +5v_US8
+3. OV_MOTOR: O ERE 1 2| oute ouTe a2t +2. BY_MSMP2
+2. 8V_RX0<T~ outs - Fov 2L USB_AX_DATA
e I N gl & 2 8 o2 T +2. 6V_MSMA
12.8v_Axt ° IHEITE ala il 8y Yanl, YgmeonT [ -
A 205 IR FEERE ER AL =Rl e |+ . | i
ancaw By EEEEEEEEEE Al 1o mI b I
TCSCHLALTEMIAR o g 143 T A LA - <JusE_vMO
noi2 o “ d + <7 USB_VFO
&7 ERE El t < +vPwA
NS 1008 R B 1. 8Y_MEMP1
+2. Bv-LCO
AC_OK/* o
v ; 7 ro +2. BY_TX
¢ z (8 HE
H e 2 | g
e 8 g
glal lo Is 81|y
s N g
8T8 TR

oy
5
E

¥

RESIN/
BAT_ADC






[image: image48.jpg]@GND PS_HOLD

RESIN MSMP TRST TMS
TDO RCLK TCK

VX8600
86V7V02-08

1

S

PEETTTITCCCCTLTTTTITCES
T

L )






 SHAPE  \* MERGEFORMAT 
[image: image49]

[image: image50.png]VCTCXO(19. 2 MHZzZ)

= >

L7o2

150
+2. 8V_TCX0 > T
A715
51 3518 Lc7is
1855 R88 = 9% -
a= 1005 n| &
7oz e gLE
o
Ao ToTsHoaRU 11005 “l
+oc 1 INC vCC
1005 A . o
TAK_LO_ADJ > T ve  our ™ e
s
166 Lozin| ono aND ouTY T MEMLTEXO
ion X700
K o BT_TCXO
1505
= TCXO
uros
A1t 1402810-FA2F
s s F7is
VPN By Voo ce 5—< TCXO_EN
ey 2lenp1 oDl S
~3 s . A
o= VOUT NC|

A720
o






[image: image51.jpg]








[image: image52.png]





[image: image53.jpg]232ex% 10 L}

I










 SHAPE  \* MERGEFORMAT 
[image: image54]


 SHAPE  \* MERGEFORMAT 

[image: image55]


[image: image56.png]:1%“'“.







[image: image57.jpg]










[image: image58.png]cis1 mul cio7 99 Tides oo Tisoe
SooaT @8 e gl E=A
,ooslgal 1608
98 l
9
8
£

LEEEREEEREE Tes
T 12819883183 @ &4
B | EsERagsiig 1 1
] 33 celio Na_IN om0
32 celLo_LNA_BIAS mxOaM [ Z—
24 peso_LNA_IN o8 ——
a 0| PCSO-LNA-BIAS Aximm B
13 Gps_PLNA_IN [T e ——
GPS_PLNA_BIAS oM
oo 42— T &
— S& cEloNaQuT croOUT 44—
18 gps_1m cpavaniee T 4

19 gpa NP caour (A —
4| e Lo zam GVCO-TUNE Lgﬂ@ss—giégz—c GYCO-T

— 2 CELLO_INP Toxo snz Texo
sesto (48502 4 5pgTo
8 25 poso-INM sBoTO/SSBI (22— <7 SBDTO
e b3 PCSO_INP sacKo (Pl ————————————<71 SACKO
= sBSTL[P4 <7 SAST1
o, | ceELLI-IN saoms < sBoT1
Bpa iAs smck f%?ééf—c SBCKL

rEseRven B4 157
oo o
3

Aj. 51 s 1




4.2 TX Part Trouble
4.2.1 DCN TX Trouble 
[image: image59.jpg]232ex% 10 L}

I




 

Checking Flow 



[image: image60.emf]  1.Check

    LDO Circuit

START

vPress “MENU” +“0000000” in Phone idle state

vPress “8”FCC Test

vPress “2” CDMA FCC

-Set CH384 & AGC360

vSpectrum analyzer setting

vOscilloscope setting

2.Check VCTCXO

  Circuit

3.Check RFT6150

   Check

4. Check RF SAW

5.Check Dual

    PAM

6.Check Duplexer

7.Check Triplexer

8.Check Mobile

   Switch

Re-download, CAL


4.2.1.1. Check Regulator Circuit 
TEST POINT 


[image: image61.png]



TX_LDO_EN
   < +2.8V_TCXO >

Circuit Diagram                           Checking Flow
[image: image62.png]F2. 8V_TCXO

[ +2. 8V _TCXO!

+VPWR >

uz01
A1114D2B1D-TA-F

1

A718

T Voo cef6 5—<1 TCXO-EN
& 2lann1 enpelS

I s M

5 vour  ng




[image: image63.png]i

START

Check 1 Pin3

NO
Replace U701

YeS

LDO Gircuit s OK Soce
Next Page





Test Point                           Checking Flow

[image: image64.png]





Circuit Diagram
[image: image65.png]KVCTCXO(19. 2 MHz) & BUFFER>

+2.8Y_TCX0 D>

TAKLO-ADS O





TCXO Waveform
 SHAPE  \* MERGEFORMAT 




TEST Point 


[image: image67.jpg]232ex% 10 L}

I






Circuit Diagram

[image: image68.png]VPR

PRV a e e
et ol ] thpe [Len LTX PART
ST | T TR g 2>
o ] ]
EEEREETEEE
H 9453393
EEEEEHEEL a8 @9
- an o LD
oy b SIS
o s i oSl
e mm  omcon
o P i
T oo [T o
mer (A< sa5T0
oL o B 5A0KD

Lock DET











Test Point                          Checking Flow
[image: image69.jpg]232ex% 10 L}

I








[image: image70.png]856579

ouT

G3

GaG1

IN

5

3 2










[image: image71.jpg]232ex% 10 L}

I




 





[image: image72.png]2
el glmg lcm PA_ONC
’W}T Bral T,
®
8
de
ml
n T
S
" 13 Egl
\ [
f [ —
a3
5] vece vees [ 35z
To| veoaa wMoDE (5 g
TIRFOUTCELL  RFIN.CELL [ ,
$—g onoe VREF_CELL [T
AnTE10R —
7a02 o Oio7 o
5
B
i
PA_A1CE—$
a
o o
L dy
5

I








[image: image73.jpg]232ex% 10 L}

I






Circuit Diagram                Waveform



[image: image74.png]2| L M Jd4

7929E9 795985

XL X
v

Dag
cotan





Graph 4.2.1.8

4.2.1.8 Chcek Triplexer

Test Point                              Checking Flow
[image: image75.jpg]232ex% 10 L}

I




    
[image: image76.emf]       Similar?

START

Check U105 Pin7,2

CELL_PORT

ANT_PORT

Refer to Graph 4.2.1.9

U105

Check U105 Pin 7

Triplexer Circuit is

YES

NO

        >=2.5

Replace U102

Check CDMA_MODE

Signal Line & MSM

NO

YES




Circuit Diagram                            Waveform

[image: image77.png]1005

<138

u101
01034

pcs |8

anT CELL

oPs
518233640566

EEEEE







4.2.1.9 Chcek Mobile Switch
Test Point                            Checking Flow
[image: image78.jpg]232ex% 10 L}

I




      
[image: image79.emf]     Similar?

START

Check U107 Pin1,2

RF_IN

RF_OUT

Refer to Graph 4.2.1.10

U407

Replace U107

Mobile Switch is 

OK

YES

NO



 [image: image80.png]


 




[image: image81]
  
        








[image: image82.emf]


TX_LDO_EN


[image: image83.png][+2. 8v_TCX0O]

P " T roro
B &

42 Ev,Tcxucl—Wv—ﬁ








[image: image84.emf]



Circuit Diagram
[image: image85.png]KVCTCXO(19. 2 MHz) & BUFFER>

+2 mv_ToO -

TAcLo a0 D>








 [image: image86.png]wal - Microsoft Word

ice

VXBEOD_

T BEE W SED ANQ ERHD EA W SsEW EEEEEEEIE g
DEEHRS @RIV B9 c 8 d0E MCIEER) )
212 heac + HEH - | 7 2t~ 9 5P CAr AN 1c) ") A =
T T e e o TR T I DT I DT TRRETRE TR TRT TRRT TNT T TRT TRNT TRRT ]
4.2.1.4. Check RFT Circuit. .
PIN 27TX_QPw
N PIN 28TX_QM-
TEST Point . PIN 29TX_IPv
PIN 30TX_IM-
P
P
P
TX_ONw
SBCKO-
SBDTO-
SBSTO.
P
Circuit Diagram. o
af@]= o< >
W@k [ERW-NNDOOHB S @RS s-A-==5adf
|
B HON 239 64/l2l ol E 2 |[JI=| 55T @3 930 | %




Circuit Diagram
[image: image87.png]











         
[image: image88.jpg]232ex% 10 L}

I







[image: image89.jpg]







[image: image90.png]DEE BIE =AW SB0O ANNOQ) =D EG BW =8

VREF_CELL-
RF_IN- veet.

RF_OUTPUT. PA_R1.

Check U112 Pin5, 8.
RF_IN /RF_OUT
Refer to Graphé.2.1.6.

o
YES-
PAM Circuit is OK

NO.!

YES-

NO ™1 Check +2.9v_TX Line.
And PA_ON-

YES

Check VCC Line-

“«o g

68 HOIX 2 3% 68/120 Il E =









[image: image91.png]~7402

] Idn
13
asus
|
| FFouT_Pos VREF_PUS [5
5] enat RFIN_PCS [
5] veee vees [
1d] veo2A vvoDE [
TIFFOUT_CELL  RFINCELL [
g snoe VREF_CELL [T
AWTEI10R
> Uto7
PA_R1CE—$
B

I







[image: image92.jpg]232ex% 10 L}

I







[image: image93.png]oP101
AcMD-7402
Lisp x
By L
o v
H
H 6
x
L113 3
oNI
1005
+2.9v_TX
.





[image: image94.jpg]232ex% 10 L}

I






[image: image95.png]1005

Liol

sén
9]
58
5 u101
01034
pcs |8
21 ant CELL

oPs
518233640566

EEEEE

cioal

L1o4

R Sy

A102
15






[image: image96.jpg]232ex% 10 L}

I








[image: image97.png]







[image: image98.png]50






[image: image99.png]


 





[image: image100]
        [image: image101.png]


  
[image: image102]

[image: image103.png]<BATTEAY CONTACT> .







[image: image104]

[image: image105.png]outs - <JsLCDCs/
am e Lo pesET/
o a2 Sves/
; ; Agaurs  zuais <IMLCO_CS/
= 1 Llam e <aLCD_Re/
Anwo Bloms e[S <1 OE2/
1o om0 +2.8v_Lc0 D——4— o [ = o
2 olie s = o rovdanamesamomadarn
25 50 W 2 = EEETIT e
4l oura|E & = Slora el G pelal
. an TS| ] oura a2 < pal10]
3 ; 1| Sloura  ana[ < pal11]
00141 Do aalg) s Ha k
[T =] g L [ o
D2l6] o= ovrzfg VMED Dy T Gors  me— pelie]
0el7) 2o s LEDs: 2 < 2,04 Hoors  wa oel1a
‘ LEDRTH afoura  me[z—<3 D2l14]
LEDa<H < Sfourt  mafs—<3 oalsl
P e r— 1
HEE gp; LR J ~> Loo_ o creac






[image: image106.png]












[image: image107.png]<CAUD

22
SPK_EN &> I
AR usos +
o LmagzaLax g
b g
&
SPK_IN- > | jg520 s - BYp (& o
os1a! FERo Ny e 525 100p
SPK_INT D HERASWY . Bt o5 I8
83 Gobb i00p
> 7 cau|
Al 9]
VPR > &
@
la
a8 RS13
T8 B2k

559
4

SPK+

SPK—





[image: image108.png]





[image: image109.png]CANALOG

usos
NLASS2238MNR2E

1

SWo

cs21
i

I4 —t

<] +2. BY_MSMA

EAR+ >

cams  voo

s

L503 (FTHB-1M1005-501JT

RCV+H

EAR- &>

camz  NC1
N1 Ne2

7

L502 (1) HB=1M1005-60 1T

> RCV—

SW_EN B>

LDNT

csa7,

ceg ot

> SPK_INt

w2 NoL

10

> SPK_IN—

E}
)
4
]

f oD NDZ





 [image: image110.png]








4.3.3.3 Mic Trouble

[image: image111.png]vasToev

YOS, /ONAS WOV
~OVOS,/NIG-HIG—XNY.
OVO,/1NDA WA XY
~OVOS,/A10HOE XY

aWooD
svIEazK
Noxnw
aoxnv
NOTEv3
orvs
NELOTW
80T
NNINIW
aNTOTW
NLNONTH
a1mmaTn
NEXNY
arxny
NZazR
20Th
NIOIW
STIIH

10TE9/N-0FENTSADY
|DTdE/NF NS AT
|DTan/NEEENTSATY
10TC9/NEFENISADY
|DTdE/NpIENTEATY

(26 101e8/OHINAS
(¥ 101D/ FHINAS
(59101 aD,/EHINAS

e >/INI W3
fes————> /N3 8sn

E——>@ 103135 0av

per L= VUSWvwa

i SHOMLIN OIW

L0
pemv— %
fezme——>8VI870Il
[prov

T

L

levov b3
> +av3
=

EESs

510K
R413

ot ' v
Tr  Eive

M-
180¢
RA410

are)
i)

I
1

=

==

52y

BFav
BFav

v torvo &
erave! Fagrs <D OIW HY3

S

INC

ICH_ACK C>—————————5) utMz_cLi/ePIole7]

WR_DOWN<3———————23 utmz_ReseT/GrIols6]
- BL_EN<IF———2% Utz pwm_en/erPIoles]

SENSE/ 13 Utmz naTa/GPIO 841

sov5
-
& o ovo sovts
S
E— Au\m_mﬂmmunm No P rors; -
a2
[E10AN
(1T
[£]0dA% J 7
37037 30Is 5 7
ol 5 8
NI~vHINYD 84| ° &
/LIST WD o] . g
16w
gl ey
52 &
Bl 7’m
st
P rer e I
S
&5
u$
at
20vg
i
[ —
a
4
It
a
J
Q
5





[image: image112.png]—

oo o ps_now
©00 0 resin psw st Tus
0000 ™ o Rkl

ovi. 0) ‘
LGE 06734







[image: image113.png]3 1% -
gy [l ¥ Cem = < +2. BV_NSHA
APV e e Ig 1 I
s T8 opsmcor

Lo Ao



[image: image114.png]<EARPHON

KEY >

+1.8V_MEMP1 w0z
i et pes

ﬁﬂ”_D EAR-MIC






[image: image115.png]



    













[image: image116.png]<EL DRIVER>

Tm_c. .
5y

MKEY_BL_END>— e

D Lo

s’






[image: image117.png]


[image: image118.png]A200020 - Windows AP 2 B4 S0

12T | PLlan|XsHE| Q@




  [image: image119.png]






[image: image120.png]PHIC_SDA>———————

+3. OV_MOTOR -




[image: image121.png]ICVE10184E150R104FR

TEEE R EEER
BEERE R RERERPPRPREFP PR

congos
Ax<7La0Z27G
CAM_SCL > L ouTs[®
2 a
CaM_s0A O ma  outz o
CAM_VSYNC C>——23{ms  outs|Z L
CAM_HSYNC > 4lna ouTalS .
oo -
B 4 & 3B
+2. BV_CAM_AVDD_I0 > s y 35
34
+1. BV_CAM_COREE> T
+3. OV_MOTOR > o232
V_M_EDC> 51
SIDE_LED_1 20
SPK+> - 29
SPK—> . 28
ACV+C> . 27
RCV—>- . 25
FKYPD[ 1] < 25
SIDE_LED_2<3 24
+3. 1V_TOUCH > 23
g 22
e | e
d
o T TT8A.T afasfazlas
3 R
8 8 8 Eg |8





[image: image122.png]


  [image: image123.png]











4.3.6  Touch Key

Test Points


[image: image124.jpg]


 [image: image125.png]= 123 - Windows AR

2

2 ma 20

iy
r

vXx8600_FPCB_REV. 10
SPCY0079501 LGE 0633

00 =&p|([Hran XxaHE|o





[image: image126.png]r Tﬂ_c +3. 1v_TouCH

L pola) Foral ﬁ
RTS 2] pal7] rars] [ pize 8
¢ puy et — 2§ FEM
e ol epm s cate 2] £
1T e R 0§ Py esTe
oy 5] esia rore 124 o= §
G joom Poto 12 e i F
I—l P1171 xres (12 "
FE885544

2 e
¥

TOUCH-CLK
TOUCH_DAT
TOUTH-ATK

























Touch key will work properly


▣ IMPORTANT

Read This Information Before Using Your Hand-Held Portable

Cellular Telephone
First introduction in 1984, the hand-held portable Cellular telephone is one of the most 

exciting and innovative electronic products ever developed.

With it you can stay in contact with your office, your home, emergency service, and others. For the safe and efficient operation of your phone, observe these guidelines.

Your Cellular phone is a radio transmitter and receiver. When it is ON, it receives and also sends out radio frequency (RF) energy. The phone operates in the frequency range of 824 MHz to 894 MHz and employs commonly used frequency modulation (FM) techniques. When you use your phone, the Cellular system handling your calls controls the power level at which your phone transmits. The power level can range from 0.006 of a watt to .6 of a watt.

▣ Exposure to Radio Frequency Energy

In 1991 the Institute of Electrical and Electronics Engineers (IEEE), and in 1992 the American National Standards Institute (ANSI) updates the 1982 ANSI Standard for safety levels with respect to human exposure to RF energy. Over 120 scientists, engineers, and physicians from universities, government health agencies, and industry, after reviewing the available boy of research, developed this updated Standard. In March, 1993, the US Federal Communications Commission (FCC) proposed the adoption of this updated Standard.

The design of your phone complies with this updated Standard. Of course, if you want to limit RF exposure even further than the updated ANSI Standard, you may choose to control the duration of your calls and opration your phone in the most power efficient manner.

▣ Efficient Phone Operation

For your phone to operate at the lowest power level, consistent with satisfactory call quality, please observe the following guidelines:

If your phone has an extendable antenna, extend it fully. Some models allow you to place a call with the antenna retracted. However, your phone operates more efficiently with the antenna fully extended.

Hold the phone as you would any other telephone. While speaking directly into the mouthpiece, position the antenna up and over your shoulder.
Do not hold the antenna when the phone is “IN USE”. Holding the antenna affects call quality and may cause the phone to operated at a higher power level than needed.

▣ Antenna Care and Replacement

Do not use the phone with a damaged antenna. If a damaged antenna comes into contact with skin, a minor bum may result. Replace a damaged antenna immediately. Consult your manual to see if you may change your antenna yourself. If so, use only a manufacture approves antenna. Otherwise, take your phone to a qualifies service center for repair.

Use only the supplied or approved antenna. Non-approved antennas, modifications, or attachments, could impair call quality, damage the phone, and violate FCC egulations.

▣ Driving

Check the laws and regulations on the use of Cellular telephones in the areas where you drive. Always obey them. Also, when using your phone while driving, please:

Give full attention to the driving. Use hands-free operation, if available, and pull off the road and park before making or answering a call if driving conditions require.

▣ Electronic Devices

Most modem electronic equipment is shielded from RF energy. However, RF energy from Cellular telephones may affect inadequately shielded electronic equipment.

RF energy may effect improperly installed or inadequately shielded electronic operating and entertainment system in motor vehicles. Check with the manufacturer or its representative to determine if these systems are adequately shielded from external RF energy. You should check with the manufacturer of any equipment that has been added to your vehicle.

Consult the manufacturer of any personal medical devices (such as pacemakers, hearing aids, etc.) to determine if they are adequately shielded from external RF energy.

Turn your phone OFF in health care facilities. When any regulations posted in the areas instruct you to do so. Hospitals or health care facilities may be using equipment that could be sensitive to external RF energy.

▣ Aircraft

Turn your phone OFF before boarding any aircraft.

Use it on the ground only with crew permission. Do not use it in the air.

To prevent possible interference with aircraft systems, US Federal Aviation Administration (FAA) regulations require you to have permission from a crew member to use your phone while the plane is on the ground. Using your phone while the plane is in the air.

▣ Children

Do not allow children to play with your phone. It is not a toy. Children could hurt themselves or others (by poking themselves or others in the eye with the antenna, for example). Children also could damage the phone, or make calls that increase your telephone bills.

▣ Blasting Areas

To avoid interfering with blasting operations, turn you unit OFF when in a “blasting area” or in areas posted “Turn off two-way radio”. Construction crews often use remote control RF devices to set off explosives.

▣ Potentially Explosive Atmospheres

Turn your phone OFF when in any area with a potentially explosive atmosphere. It is rare, but your phone or accessories could generate sparks. Sparks in such area could cause an explosion or fire resulting in bodily injury or even death.

Areas with a potentially explosive atmosphere are often, but not always, clearly marked. They include fueling areas such as gas station; below deck on boats; fuel or chemical transfer or storage facilities; areas where the air contains chemical or particles, such as grain, dust, or metal powders; and any other area where you would normally be advised to turn off your vehicle engine.
Do not transport or store flammable gas, liquid, or explosives in the compartment of your vehicle which contains your phone or accessories.

Vehicles using liquefied petroleum gas (such as propane or butane) must compl7y with the National Fire Protection Standard (NFPA-58). For a copy of this standard, contact the National Fire Protection Association, One Battery march Park, Quincy, MA 02269, Attn: Publication Sales Division.

Rule of Thumb: Using common sense at all times when handling, installing or using the phone. Any questions should be directed to you nearest Service Center or authorized service technician or electrician.


General Terms

Abbreviated Alert. An abbreviated alert is used to remind the mobile station user that previously selected alternative routing features are still active.

AC. See Authentication Center.

Access Attempt. A sequence of one or more access probe sequences on the Access Channel containing the same message. See also Access Probe and Access Probe Sequence.

Access Channel. A Reserve CDMA Channel used by mobile stations for communicating to the base station. The Access Channel is used for short signaling message exchanges such as call origination’s, responses to pages, and registrations. The Access Channel is a slotted random access channel.

Access Channel Message. The information part of an access probe consisting of the message body, length field, and CRC.

Access Channel Message Capsule. An Access Channel message plus the padding.

Access Channel Preamble. The preamble of an access probe consisting of a sequence of all-zero frames that is sent at the 4800bps rate.

Access Channel Request Message. An Access Channel message that is autonomously generated by the mobile station. See also Access Channel Response Message.

Access Channel Response Message. A message on the Access Channel generated to reply to a message received from the base station.

Access Channel Slot. The assigned time interval for an access probe. An Access Channel slot consists of an integer number of frames. The transmission of an access probe is performed within the boundaries of an Access Channel slot.

Access Probe. One Access Channel transmission consisting of a preamble and a message. The transmission is an integer number of frames in length and transmits one Access Channel message. See also Access Probe Sequence and Access Attempt.

Access Probe Sequence. A sequence of one or more access probes on the Access Channel. The same Access Channel message is transmitted in every access probe of an access attempt. See also Access Probe and Access Attempt.

Acknowledgement. A Layer 2 response by the mobile station or the base station confirming that a signaling message was received correctly.

Action Time. The time at which the action implied by a message should take effect.

Active Set. The set of pilots associated with the CDMA Channels containing Forward Traffic Channels assigned to a particular mobile station.

Aging. A mechanism through which the mobile station maintains in its Neighbor Set the pilots that have been recently sent to it from the base station and the pilots whose handoff drop timers have recently expired.

A-key. A secret, 64-bit pattern stored in the mobile station. It is used to generate update the mobile station’s Shared Secret Data. The A-key is used in the mobile station authentication process.

Analog Access Channel. An analog control channel used by a mobile station to access a system to obtain service.
Analog Color-Code. An analog signal (see Supervisory Audio Tone) transmitted by a base station on an analog voice channel and used to detect capture of a mobile station by an interfering base station or the capture of a base station by an interfering mobile station.

Analog Control Channel. An analog channel used for the transmission of digital control information from a base station to a mobile station or from a mobile station to a base station.

Analog Paging Channel. A forward analog control channel that is used to page mobile stations and send orders.

Analog Voice Channel. An analog channel on which a voice conversation occurs and on which brief digital messages may be sent from a base station to a mobile station or from a mobile station to a base station.

Authentication. A procedure used by a base station to validate a mobile station’s identity.

Authentication Center (AC). An entity that manages the authentication information related to the mobile station.

Authentication Response (AUTHR). An 18-bit output of the authentication algorithm. It is used, for example, to validate mobile station registrations, origination and terminations. A method of registration in which the mobile station registers without an explicit command from the base station.

AWGN. Additive White Gaussian Noise.

Bad Frames. Frames classified as erasures (frame category 10) or9600bps frames, primary traffic only with bit errors (frame category 9). See also Good Frames.

Base Station. A station in the Domestic Public Cellular Radio Telecommunications Service, other than a mobile station, used for communicating with mobile stations. Depending upon the context, the term base station may refer to a cell, a sector within a cell, an MSC, or other part of the Cellular system. See also MSC.

Base Station Authentication Response (AUTHBS). An 18-bit pattern generated by the authentication algorithm. AUTHBS is used to confirm the validity of base station orders to update the Shared Secret Data.

Base Station Random Variable (RANDBS). A 32-bit random number generated by the mobile station for authenticating base station orders to update the Shared Secret Data.

BCH Code. See Bose-Chaudhuri-Hocquenghem Code.

Busy-Idle Bits. The portion of the data stream transmitted by a base station on a forward analog control channel that is used to indicate the current busy-idle status of the corresponding reverse analog control channel. 

Call Disconnect. The process that releases the resources handling a particular call. The disconnect process beings either when the mobile station user indicates the end of the call by generating an on-hook condition or other call release mechanism, or when the base station initiates a release.

Call History Parameter(COUNT). A modulo-64 event counter maintained by the mobile station and Authentication Center that us used for clone detection.

Candidate Set. The set of pilots that have been received with sufficient strength by the mobile station to be successfully demodulated, but have not been placed in the Active Set by the base station. See also Active Set. Neighbor Set, and Remaining Set.

. See Code Division Multiple Access

CDMA Channel. The set of channels transmitted between the base station within a given CDMA frequency assignment. See also Forward CDMA Channel and Reverse CDMA Channel.

CDMA Channel Number. An 11-bit number corresponding to the center of the CDMA frequency assignment.

CDMA Frequency Assignment.  A 1.23MHz segment of spectrum centered on one of the 30KHz channels of the existing analog system. 

Code Channel. A subchannel of a Forward CDMA Channels. A Forward CDMA Channel contains 64 code channels. Code channel zero is assigned to the Pilot Channel. Code channels 1 through 7 may be assigned to the either Paging Channels or the Traffic Channels. Code Channel 32 may be assigned to either a Sync Channel or a Traffic Channel. The remaining code channels may be assigned to Traffic Channels.

Code Division Multiple Access (CDMA).  A technique for spread-spectrum multiple-access digital communications that creates channels through the use of unique code sequences.

Code Symbol.  The output of an error-correcting encoder. Information bits are input to the encoder and code symbols are output from the encoder. See Convolutional Code. 

Continuous Transmission.  A mode of operation in which Discontinuous Transmission is not permitted.  

Control Mobile Attenuation Code(CMAC).  A 3-bit field in the Control-Filler Message that specifies the maximum authorized power level for a mobile transmitting on an analog reverse control channels.

Convolution Code.  A type of error-correcting code. A code symbol can be considered as the convolution of the input data sequence with the impulse response of a generator function. 

CRC.  See Cyclic Redundancy Code.

Cyclic Redundancy Code (CRC).  A class of linear error detecting codes which generate parity check bits by finding the remainder of a polynomial division.

Data Burst Randomizer.  The function that determines which power control groups within a frame are transmitted on the Reverse Traffic Channel when the data rate is lower than 9600 bps. The data burst randomizer determines, for each mobile station, the pseudo random position of the transmitted power control groups in the frame while guaranteeing that every modulation symbol is transmitted exactly once.

DBc. The ratio(in dB) of the sideband power of a signal, measured in a given bandwidth at a given frequency offset from the center frequency of the same signal, to the total inband power of the signal. For CDMA, the total inband power of the signal is measured in a 1.23MHz bandwidth around the center frequency of the CDMA signal. 

DBm.  A measure of power expressed in terms of its ration (in dB) to one milliwatt.

DBm/Hz.  A measure of power spectral density. DBm/Hz is the power in one Hertz of bandwidth. Where power is expressed in units of dBm.
DBW.  A measure of power expressed in terns of its ration (in dB) to one Watt.

Dedicated Control Channel. An analog conrtol channel used for the transmisson of digital control information from either a base station or a mobile station.

Deinterleaving.  The process of unpermuting the symbols that were permuted by the interleaver..

Deinterleavering is performed on reveived symbols prior to decoding.

Digital Color Code(DCC). A digital signal transmitted by a base station on a forward analog control channel that is used to detect capture of a base station by an interfering mobile station.

Dim-and-Burst.  A frame in which primary traffic is multiplexed with either secondary traffic or signaling traffic.
Discontinuous Transmission (DTX).  A mode of operation in which a mobile station transmitter autonomously switches between two transmitter power levels while the mobile station is in the conversation state on an analog voice channel.

Distance-Based Registration.  An autonomous registration method in which the mobile station registers whenever it enters a cell whose distance from the cell in which the mobile station last registered exceeds a given threshold.

DTMF.  See Dual Tone Multifrequency.

Dual-Tone Multifrequency(DTMF).  Signaling by the simultaneous transmission of two tones, one from a group of low frequencies and another from a group of high frequencies. Each group of frequecies consists of four frequencies.

Eb.  The energy of an information bit.

Ec/I0.  The ratio in (dB) between the pilot energy accumulated over one PN chip period(Ec) to the power spectral density in the received bandwidth(Io).

Effective Radiated Power (ERP).  The transmitted power multiplied by the antenna gain referenced to a half wave dipole.

Electronic Serial Number(ESN).  A 32-bit number assigned by the mobile station manufacturer, uniquely identifying the mobile station equipment.

Encoder Tail Bits.  A fixed sequence of bits added to the end of a block of data to reset the convolutional encoder to a known state.

ERP.  See Effective Radiated Power.

ESN.  See Electronic Serial Number.

Extended Protocol.  An optional expansion of the signaling message between the base station and mobile station to allow for the addition of new system features and operational capabilities.

Fade Timer. A timer kept by the mobile station as a measure of Forward Traffic Channel continuity. If the Fade timer expires, the mobile station drops the call.

Flash. An indication sent on an analog voice channel or CDMA Traffic Channel indicating that the user Directed the mobile station to invoke special processing.

Foreign NID Roamer. A mobile station operating in the same system (SID) but a different network (NID)Form the one in which service was subscribed. See also Foreign SID Roamer and Roamer.

Foreign SID Roamer. A mobile station operating in a system (SID) other than the one from which service was subscribed. See also Foreign NID Roamer and Roamer.

Forward Analog Control Channel (FOCC). An analog voice channel used from a base station to a mobile station.

Forward  Analog Voice Channel (FVC). An analog voice channel used  from a base station to a mobile station. 

Forward CDMA Channel. A CDMA Channel form a base station to mobile stations. The Forward CDMA Channel contains one or more code channels that are transmitted on a CDMA frequency assignment using a Particular pilot PN offset. The code channels are associated with the Pilot Channel, Sync Channel, Paging Channels, and Traffic Channels. The Forward CDMA Channel always carries a Pilot Channel and may carry up to one Sync Channel, up to seven Paging Channels, and up to 63 Traffic Channels, as long as the total number of channels, including the Pilot Channel, is no greater than 64.
Forward Traffic Channel. A code channel used to transport user and signaling traffic from the base station to the mobile station.

 A basic timing interval in the system. For the Access Channel, Paging Channel, and Traffic Channel, a frame is 20 ms long. For the Sync Channel, a frame is 26.666…ms long.

Frame Category. A classification of a received Traffic Channel frame based upon transmission data rate, the Frame contents (primary traffic, secondary traffic, or signaling traffic), and whether there are detected error in the frame.

Frame Offset. A time skewing of Traffic Channel frames from System Time in integer multiples of 1.25 ms. The maximum frame offset is 18..75 ms..

Frame Quality Indicator. The CRC check applied to 9600 bps and 4800 bps Traffic Channel frames. 

Global Positioning System (GPS). A US government satellite system that provides location and time Information to users. See Navstar GPS Space segment / Navigation User interfaces ICD-GPS-200 for Specifications.

Half Frame. A 10 ms interval on the paging Channel. Two half frames comprise a frame, the first half frame begins at the same time as the frame.

Handoff. The of transferring communication whth a station mobile station from one base station to another.

Hard Handoff. A handoff characterized by a temporary disconnection of the Traffic Channel. Hard handoffs Occur when the mobile station is transferred between disjoint Active Sets, the CDMA frequency assignment changes, the frame offset changes, or the mobile station is directed from a CDMA Traffic Channel to an analog voice channel, See also Soft Handoff.

Hash Function. A function used by the mobile station to select one out of N available resource. The hash function distributes the available resources uniformly among a random sample of mobile stations.

HLR. See Home Location Register.

Home Location Register (HLR). The location register to which a MIN is assigned for record purposes such as subscriber information.

Home System. The Cellular system in which the mobile station subscribes for service.

Idle Handoff. The act of transferring reception of the Paging Channel from one bass station to another, when the mobile station is in the Mobile Station Idle State.
Implicit Registration. A registrationachieved by a successful transmission of an origination or page response on the Access Channel.

Interleaving. The process of permuting a sequence of symbols.

kHz. Kilohertz (103 Hertz).

ksps. Kilo-symbols per second (103 symbols per second).

Layer 1. See Physical Layer.

Layer 2. Layer 2 provides for the correct transmission and reception of signaling messages, including partial duplicate detection. See also Layering and Layer 3.

Layer 3. Layer 3 provides the control of the Cellular telephone systems. Signaling messages originate and terminate at layer 3. See also Layering and Layer 2.
Local Control. An optional mobile station feature used to perform manufacturer-specific functions.

 A PN sequence with period 242-1 that is used for scrambling on the Forward CDMA Channel and spreading on the Reverse CDMA Channel. The long code uniquely identifies a mobile station on both the Reverse Traffic Channel and the Forward Traffic Channel.  The long code provides limited privacy. The long code also separates multiple Access Channels on the same CDMA channel. See also Public Long Code and Private Long Code.

Long Code Mask. A 42-bit binary number that creates the unique identity of the long code. See also Public Long Code, Private Long Code, Public Long Code Mask, and Private Long Code Mask.

LSB. Least significant bit.

Maximal Length Sequence (m-Sequence). A binary sequence of period 2n-1, n a positive integer, with no internal periodicities. A maximal length sequence can be generated by a tapped n-bit shift register with linear feedback.

Mcps. Megachips per second (106 chips per second).

Mean Input Power. The total received calorimetric power measured in a specified bandwidth at the antenna connector, including all internal and external signal and noise sources. 

Mean Output Power. The total transmitted calorimetric power measured in a specified bandwidth at the antenna connector when the transmitter is active.

Message. A data structure that conveys control information or application information. A message consists of a length field (MSG_LENGTH), a message body (the part conveying the information), and a CRC.

Message Body. The part of the message contained between the length field (MSG_LENGTH) and the CRC field.

Message Capsule. A sequence of bits comprising a single message and padding. The padding always follows the message and may be of zero length.

Message CRC. The CRC associated with a message. See also Cyclic Redundancy Check.

Message Field. A basic named element in a message. A message field may consist of zero or more bits.

Message Record. An entry in a message consisting of one or more field that repeats in the message.

MHz. Megahertz.(106 Herz)

MIN. See Mobile Station Identification Number.

Mobile Protocol Capability Indicator (MPCI). A 2-bit field used to indicate 속 mobile station’s capabilities.

Mobile Station. A station in the Domestic Public Cellular Radio Telecommunications Service intended to be used while in motion or during halts at unspecified points. Mobile station include portable units (e.g., handheld personal units) and units installed in vehicles.

Mobile Station Class. Mobile station classes define mobile station characteristics such as slotted operation and transmission power. 

Mobile Station Identification Number (MIN). The 34-bit number that is a digital representation of the 10-digit directory telephone number assigned to a mobile station.

Mobile Station Originated Call. A call originating from a mobile station.

Mobile Station Terminated Call. A call received by a mobile station (not to be confused with a disconnect or call release).

Mobile Switching Center (MSC). A configuration of equipment that provides Cellular radiotelephone service. Also called the Mobile Telephone Switching Office (MTSO)

Modulation Symbol. The output of the data modulator before spreading. On the Reverse Traffic Channel, 64-ary orthogonal modulation is used and six code symbol (when the data rate is 9600bps) or each repeated code symbol (when the data rate is less than 9600bps) is one modulation symbol.

Ms. Millisecond.

MSB. Most significant bit.

MSC. See Mobile Switching Center.

Multiplex Option. The ability of the multiplex sublayer and lower layer to be tailored to provide special capabilities. A multiplex option defines such characteristics as the frame format and the rate decision rules. See also Multiplex Sublayer.

Multiplex Sublayer. One of the conceptual layers of the system that multiplexes and demultiplexes primary traffic, secondary traffic, and signaling traffic.

NAM. See Number Assignment Module.

Narrow Analog. A type of voice channel that uses 10kHz channel spacing and subaudible signaling.

Neighbor Set. The set of pilots associated with the CDMA Channel that are probable candidates for handoff. Normally, the Neighbor Set consists of the pilots associated with CDMA Channel that cover geographical areas near the mobile station. See also Active Set, Candidate Set, and Remaining Set.

· A network is a subset of a Cellular system, such as an area-wide Cellular network, a private group of base stations, or a group of base stations set up to handle a special requirement. A network can be as small or as large as needed, as long as it is fully contained within a system. See also System.

Network Identification (NID). A number that uniquely identifies a network within a Cellular system. See also System Identification.

NID. See Network Identification.

Non-Autonomous Registration. A registration method in which the base station initiates registration. See also Autonomous Registration.

Non-Slotted Mode. An operation mode of the mobile station in which the mobile station continuously monitors the Paging Channel when in the Mobile Station Idle State.

Ns. Nanosecond.

NULL. Not having any value.

Null Traffic Channel Data. One or more frames of 16 ‘1’s followed by eight ‘0’s sent at the 1200bps rate. Null Traffic Channel data is sent when no service option is active and no signaling message is being sent. Null Traffic Channel data serves to maintain the connectivity between the mobile station and the base station.

Number Assignment Module (NAM). A set of MIN-related parameters stored in the mobile station.

Numeric Information. Numeric information consists of parameters that appear as numeric fields in message exchanged by the base station and the mobile station and information used to describe the operation of the mobile station.

OLC. See Overload Class (CDMA) or Overload Control (analog).

Optional Field. A field defined within a message structure that is optionally to the message recipient.

Order. A type of message that contains control codes for either the mobile station or the base station.

Ordered Registration. A registration method in which the base station orders the mobile station to send registration related parameters.

Overhead Message. A message sent by the base station on the Paging Channel to communicate base-station-specific and system-wide information to mobile station.

Overload Class. The means used to control system access by mobile stations, typically in emergency or other overload conditions. Mobile station are assigned one (or more) of sixteen overload classed, Access to the CDMA system can then be controlled on a per class basis by persistence values transmitted by the base station.
Overload Control (OLC). A means reverse analog control channel accesses by mobile stations. Mobile station are assigned one(or more) of sixteen control levels. Access is selectively restricted by a base station setting one or more OLC bits in the Overload Control Global Action Message.

Packet. The unit of information exchanged between the service option applications of the base station and the mobile station.

Padding. A sequence of bits used to fill from the end of a message to the end of a message capsule, typically to the end of the frame or half frame. All bits in the padding are '0'.

Paging. The act of seeking a mobile station when a call has been placed to that mobile station.

Paging Channel (Analog). See Analog Paging Channel.

Paging Channel (CDMA). A code channel in a Forward CDMA Channel used for transmission of control information and pages from a base station to a mobile station.

Paging Channel Slot. An 80ms interval on the Paging Channel. Mobile station operating in the slotted mode are assigned specific slots in which day monitor messages from the base station.

Parameter-Change Registration. A registration method in which the mobile station registers when certain of its stored parameters change.

Parity Check Bits. Bits added to a sequence of information bits to provide error detection, correction, or both.

Persistence. A probability measure used by the mobile station to determine if it should transmit in a given Access Channel Slot.

Physical Layer. The part of the communication protocol between the mobile station and the base station that is responsible for the transmission and reception of data. The physical layer in the transmitting station is presented a frame by the multiplex sublayer and transforms it into an over-the-air waveform. The physical layer in the receiving station transforms the waveform back into a frame and presents it to the multiplex sublayer above it.

Pilot Channel. An unmodulated, direct-sequence spread spectrum signal transmitted continuously by each CDMA base station. The Pilot Channel allows a mobile station to acquire the timing of the Forward CDMA Channel, provides a phase reference for coherent demodulation, and provides a means for signal strength comparisons between base station for determining when to handoff.

Pilot PN Sequence. A pair of modified maximal length PN sequences with period 215  used to spread the Foward CDMA Channel and the Reserve CDMA Channel. Different base station are identified by different pilot PN sequence offsets.

Pilot PN Sequence Offset Index. The PN offset in units of 64 PN chips of a pilot, relative to the zero offset pilot PN sequence.

PN Chip. One bit in the PN sequence.

PN Sequence. Pseudonoise sequence. A periodic binary sequence.

Power Control Bit. A bit sent in every 1.25ms interval on the Forward Traffic Channel to signal the mobile station to increase or decrease its transmit power.
Power Control Group. A 1.25ms interval on the Forward Traffic Channel and the Reverse Traffic Channel.

See also Power Control Bit.

Power-Down Registration. Au autonomous registration method in which the mobile station registers on power up.

PPM. Parts per million.

Preamble. See Access Channel Preamble and Traffic Channel Preamble.

Primary CDMA Channel. A CDMA Channel at a pre-assigned frequency assignment used by the mobile station for initial acquisition. See also Secondary CDMA Channel.

Primary Paging Channel (CDMA). The default code channel (code channel 1) assigned for paging on a CDMA Channel.

Primary Traffic. The main traffic stream carried between the mobile station and the base station, supporting the active promaty service option, on the Traffic Channel. See also Secondary Traffic, Signaling Traffic, and Servic3e Option.

Private Long Code. The long code characterized by the private long code mask. See also Long Code.

Private Long Code Mask. The long code mask used to form the private long code. See also Public Long Code Mask and Long Code.

Public Long Code. The long code characterized by the public long code mask.

Public Long Code Mask. The long code mask used to form the private long code. The mask contains the ESN of the mobile station. See also Private Long Code Mask and Long Code.

Punctured Code. An error-correcting code generated from another error-correcting code by deleting (i.e., puncturing) code symbols from the code output.

Quick Repeats. Additional transmissions of identical copies of a message within a short interval to increase the probability that the message is received correctly.

Receive Objective Loudness Rating (ROLR). A perceptually weighted transducer gain of telephone receivers relating electrical excitation from a reference generator to sound pressure at the earphone. The receive objective loudness tating is normally specified in dB relative to one Pascal per millivolt. See IEEE Standard 269-1992, IEEE Standard 661-1979, CCITT Recommendation P.76, and CCITT Recommendation P.79.

Registration. The process by which a mobile station identifies its location and parameters to a base station.

Registration Zone. A collection of one or more base stations treated as a unit when determining whether a mobile station should perform zone-based registration.

Release. A process that the mobile station and base station use to inform each other of call disconnect.

The set of all allowable pilot offsets as determined by PILOT_INC, excluding the pilot offsets of the pilots in the Active Set, Candidate Set, and Neighbor Set. See also Active Set, Candidate Set, and Neighbor Set.

Request. A layer 3 message generated by either the mobile station or the base station to retrieve information, ask for service, or command an action.
Response. A layer 3 message generated as a result of another message, typically a request.

Reverse Analog Control (RECC). The analog control channel used from a mobile station to a base station.

Reverse Analog Voice Channel (RVC). The analog voice channel used from a mobile station to a base station.

Reverse CDMA Channel. The CDMA Channel from the mobile station to the base station. From the base station’s perspective, the Reverse CDMA Channel is the sum of all mobile station transmissions on a CDMA frequency assignment.

Reverse Traffic Channel. A Reverse CDMA Channel used to transport user and signaling traffic from a single mobile station to one or more base stations.

Roamer. A mobile station operating in a Cellular system (or network) other than the one from which service was subscribed. See also Foreign NID Roamer and Foreign SID Roamer.

ROLR. See Receive Objective Loudness Rating.

SAT. See Supervisory Audio Tone.

Scan of Channels. The procedure by which a mobile station examines the signal strength of each forward analog control channel.

SCI. Synchronized Capsule Indicator bit.

Search Window. The range of PN sequence offsets that a mobile station searches for a pilot.

Secondary CDMA Channel. A CDMA Channel at a preassigned frequency assignment used by the mobile station for initial acquisition. See also Primary CDMA Channel.

Secondary Traffic. An additional traffic stream that can be carried between the mobile station and the base station on the Traffic Channel. See also Primary Traffic and Signaling Traffic.

Seizure Precursor. The initial digital sequence transmitted by a mobile station to a base station on a reverse analog control channel.

Seizure Option. A service capability of the system. Service options may be applications such as voice, data, or facsimile.

Shard Secret Data (SSD). A 128-bit pattern stored in the mobile station (in semi-permanent memory) and known by the base station. SSD is a concatenation of two 64-bit subsets: SSD_A, which is used to support the authentication procedures and SSD_B, which serves as one of the inputs to the process generating the encryption mask and private long code.

Short Message Services (SMS). A suite of services which include SMS Text Delivery, Digital Paging (i.e., Call Back Number – CBN), and Voice Mail Notification (VMN).

SID. See System Identification.

Signaling Tone. A 10kHz tone transmitted by a mobile station on an analog voice channel to: 1) confirm orders, 2)signal flash requests, and 3) signal release requests.

Signal Traffic. Control message that are carried between the mobile station and base station on the Traffic Channel. See also Primary Traffic and Secondary Traffic.
Slot Cycle. A periodic interval at which a mobile station operating in the slotted monitors the Paging Channel.

Slotted Mode. An operation mode of the mobile station in which the mobile station monitors only selected slots on the Paging Channel when in the Mobile Station Idle State.
Soft Handoff. A handoff occurring while the mobile station is in the Mobile Station Control on the Traffic Channel State. This handoff is characterized by commencing communications with a new base station on the same CDMA frequency assignment before terminating communications with the old base station. See also Hard Handoff.

SOM. Start-of-Message Bit.

SPS. Symbols per second.

- An identification of certain characteristics of a mobile station. Classes are defined in Table 2.3.3-1.

Status Information. The following status information is used to describe mobile station operation when using the analog system.

· Serving-System Status. Indicates whether a mobile station is turned to channels associated with System A or System B.

· First Registration ID Status. A status variable used by the mobile station in association with its processing of received Registration ID messages.

· First Location Area ID Status. A status variable used by the mobile station in association with its processing of received Location Area ID messages.

· Location Registration ID Status. A status variable used by the mobile station in association with its processing of power-up registration and location-based registration.

· First Idle ID Status. A status variable used by the mobile station in association with its processing of the Idle Task.

· Local Control Status. Indicates whether a mobile station must respond to local control messages.

· Roam Status. Indicates whether a mobile station is in its home system.

· Termination Status. Indicates whether a mobile station must terminate the call when it is on an analog voice channel.

Supervisory Audio Tone (SAT). One of three tones in the 6 kHz region that is transmitted on the forward analog voice channel by a base station and transponder on the reverse analog voice channel by as mobile station.

Supplementary Digital Color Code (SDCC1, SDCC2). Additional bits assigned to increase the number of color codes from four to sixty four, transmitted on the forward analog control channel.

Symbol. See Code Symbol and Modulation Symbol.

Sync Channel. Code channel 32 in the Forward CDMA Channel which transports the synchronization message to the mobile station.

Sync Channel Superframe. An 80ms interval consisting of three Sync Channel frames (each 26.666…ms in length).

System. A system is a Cellular telephone service that covers a geographic area such as a city. Metropolitan region, country, or group of countries.  See also Network.

System Time.  The time reference used by the system. System Time is synchronous to UTC time(except for leap seconds) and used the same time origin as GPS time. Offset by the propagation delay from the base station to the mobile station. See also Universal coordinated Time.

Timer-Based Registration.  A registration method in which the mobile station registers whenever a counter reaches a predetermined value. The counter is incremented an average of once per 80 ms period.
Time Reference.  A reference established by the mobile station that is synchronous with the earliest arriving multipath component used for demodulation.

TOLR.  See Transmit Objective Loudness Rating.

Traffic Channel.  A communication path between a mobile station and base station used for user and signaling traffic. The term Traffic Channel implies a Forward Traffic Channel and Reverse Traffic Channel pair. See also Forward Traffic Channel and Reverse Traffic Channel.

Traffic Channel Preamble.  A sequence of all-zero frames that is sent at the 9600 bps rate by the mobile station on the Reverse Traffic Channel. The Traffic Channel preamble is sent during initialization of the Traffic Channel. 

Transmit Objective Loudness Rating(TOLR).  A perceptually weighted transducer gain of telephone transmitters relation sound pressure at the microphone to voltage at a reference electrical termination. It is normally specified in dB relative to one millivolt per Pascal.  See IEEE Standard 269-1992, IEEE Standard 661-1979, CCITT Recommendation P.76 , and CCITT Recommendation. P.79

Unique Challenge-Response Procedure.  An exchange of information between a mobile station and a base station for the purpose of confirming the mobile station’s identity. The procedure is initiated by the base station and is characterized by the use of a challenge-specific random number(i.e., RANDU) instead of the random variable broadcast globally(RAND).

Unique Random Variable(RANDU).  A 24-bit random number generated by the base station in support of the Unique Challenge-Response procedure.

Universal Coordinated Time(UTC).  An internationally agreed-upon time scale maintained by the Bureau International de l’Heure(BIH) used as the time reference by nearly all commonly available time and frequency distribution systems i.e., WWW, WWVH, LORAN-C, Transit, Omega, and GPS.

UTC. Universal Temps Coordine. See Universal Coordinated Time.

Voice Channel.  See Analog Voice Channel.

Voice Mobile Attenuation Code(VMAC). A 3-bit field in the Extended Address Word commanding the initial mobile power level when assigning a mobile station to an analog voice channel.

Voice Privacy.  The process by which user voice transmitted over a CDMA Traffic Channel is a afforded a modest degree of protection against eavesdropping over the air.

Walsh Chip.  The shortest identifiable component of a Walsh function.  There are 2N  Walsh chips in one Walsh function where N is the order of the Walsh function. On the Forward CDMA channel one Walsh chip equals 1/1.2288MHz, or 813.802…ns. On the Reverse CDMA Channel, one Walsh chip equals 4/1.2288MHz, or 3.255…㎲.

Walsh Function. One of 2N time orthogonal binary functions (note that the functions are orthogonal after mapping ‘0’ to 1 and ‘1’ to –1).

Zone-Based Registration. An autonomous registration method in which the mobile station registers whenever it enters a zone that is not in the mobile station’s zone list.

㎲. Microsecond



    1. Assembly and Disassembly Diagram 

    2. Block & Circuit Diagram

    3. Component List


3.1 Accessories


3.2 Board


3.3 Mechanical Part


3.4 The Top of the Main_PCB


3.5 The Bottom of the Main_PCB

[image: image127.emf] 

F60 9


                     


                                SERVICE MANUAL








































































































 General Introduction


























PIN 49











U107

















The vibrator will


Re Assemble Main/Sub Board





Y











PCS_Duplxer





DCN_Duplxer





PIN 4 DC











No





 Check D600 (3.1V)




















4.1.1 DCN RX































































































4.1 Rx Part Trouble

















 CHAPTER 6. Glossary














 CHAPTER 5. Safety














SETTING : “on” at Motor test of test mode





Checking Flow








D600Q402

















START


























































































































 Chapter1. System Introduction
































































































































Appendix

































































R202








Test Points





4.3.5 Vibrator








Circuit Diagram





Checking Flow





No











Yes











Yes














 CHAPTER 4. Trouble Shooting













































































No





Replace EL Sheet and  ReContact Sheet





Yes











No














U109











U101





F101





DP100





F101





DP101





X700





U110





F102





U107





F103




















 CHAPTER 3. Circuit Description








L203





19.2MHz








No





Is the voltage level


VBAT 3.3~4.2V?





Check R202(3.1V)





Operate Well





Replace EL Driver

















Replace MSM

















No





Yes





Checking   R700


(EL ON Signal)





Checking   D700


(EL OUT Signal)





 R711





D700





R700





EL OUT+





GND





EL SHEET





Test Points





4.3.4 Backlight





4.3.4.1 Key Backlight





Circuit Diagram





4





Yes





Yes





Does it work well?





CHAPTER 2. NAM Input Method


(Inputting of telephone numbers included)








Change main board





No





Download SW





Level of pin2 of CON302


Is 1.65~2.1V?





No





Does it work?





Yes





No





Re-solder


R331,R332





3





Headset sending 


path problem 





4





Yes





Headset sending path problem





No





Yes





Check soldering


    U502





Re-solder Q408





Level of


EAR_JACK_SENSE/ Is low?





Download SW





No





Level of pin2 of 


CON302 Is 1.8V?





No	No





Yes	Yes





Check soldering R331,R332





Re-solder R331,R332





Headset detect problem





Headset detect problem





1





Yes





Re-soldering of


Pin4,5 of CON302





No





Change Sub board





Yes





Check soldering of


Pin4,5 of CON302





2





Checking El Sheet Contact Point (EL+,GND)





Change Main Board





Headset receiving path problem








Yes





Yes





Yes





Headset detect problem





No





The headset will work properly





4





3





Headset sending path problem





2





No





No





Can you hear the sound from the headset?





Headset receiving


Path problem





Can you hear your voice from the headset?





Change the headset and try again.





Set the audio part of the test equipment to continuous wave mode





Can you hear your voice from the headset?





Yes





Set the audio part of the test equipment to echo mode





No





1





Does the audio


Profile of the phone change to the headset mode?





Insert the headset to the handset.





START





Yes





Checking Flow





R502





U500





C512





R504





R505





R506








U502








START














4.3.3.4 Headset Trouble





Test Points





Circuit Diagram





SETTING : After initialize 5515C, Test Cellular








Yes	





Yes





MIC will work properly





No





Re-solder  MIC Network


















































er MIC NETWORK





Yes





Check the soldering 


Of MIC Network





Re-solder C407





Yes





Check the soldering 


Of C407





No





Replace the microphone





Yes	





Check the signal level


at each side of MIC.l





Replace MSM





No





Re-solder R407





No





Check the soldering


Of R407





Check the voltage level


at R407 is 1.2~1.5V








START





Checking Flow





No











Test Points





M400





C407








R407





Circuit Diagram











Checking Flow





Receiver will work properly





Yes





Check the state


Of contact of receiver





Yes








Re-solder U505





Yes





Replace U505





NO





Check the Soldering of


U505U401





NO





Check the audio signal at


L503,L502





Yes








Re-solder C527,C522





Yes





C527,C522	Replace MSM





C527,C522





NO





Check the Soldering of





NO





Check the audio signal at





Yes














START








C527





Test Points





U503(AUDIO AMP)





C522  L502,L503











4.3.3.2 Receiver Trouble





Circuit Diagram





Checking Flow





Speaker will work properly





Yes





Check the state


Of contact of speaker





Yes





Replace U503     U503401





NO





Check the Soldering of


U503U401





NO





Check the audio signal at


C528,C529





Yes





C517,C518C401, C438





NO





Check the Soldering of





NO





Check the audio signal at





Yes





CHECKING VBAT





NO





Voltage at C519


Is about 3.4~4.2V





START





Yes








Re-solder U503401





Yes








Re-solder cCC517,CC401, C438





C517,C518	Replace MSM





SETTING : “Melody on” at sounds of test menu.











C517





C529








C518





C528





U505(ANALOG SWITCH)





Test Points











Circuit Diagram





4.3.3 Audio





4.3.3.1 Speaker Trouble





LCD display will work properly





Yes





No


Check CON502 Soldering	Re-solder CON502





Check the


Control signal Data[0:15] LCD_RESET/, LCD_CS/, WR/





; Check F601,F602, F609, F608, F607 ( Signal line )





Yes





Yes





Yes





No





Replace U201





Voltage of R240==2.8V?





START





No


Does LCD work properly                        Change the LCD Module





Checking Flow





Test Points





• Check point





- The assembly status of the LCD Module


- The assembly status of the main connector


- The Soldering of connector





4.3.2 LCD





4.3.2.1 LCD Trouble





Circuit Diagram





• LCD Control signals


From MSM : D[0:15], LCD_RESET/, MLCD_CS/, OLED_ON, RS, WE2/





YES





NO





YES





NO





Check the soldering Q701





Re-solder U201





Replace U201





Voltage


of pin 2 CON701 == VBATT





YES





Charging will operate properly





The battery may have the problem.


Change the battery and try again.





NO





Is the battery


charged





Replace U201





YES





NO





NO





YES





YES





0.1V < Voltage of CHAR_MON< 1.4V





Voltage at +V_CHAR


of U201 = 5V?





The charging adaptor (TA) is out of order.


Change the charging adaptor.





Re-soldering CON302





NO





 Charging Connector CON302


Well-soldered?





YES





START





Test Points





U405





U410





Q409





Checking Flow





Circuit Diagram	Test Points





VBATT(+VPWR)





V_CHAR





F601 -> D2[0].D2[1].D2[2].D2[3].





CON302





4.3.1.2 Charging Trouble





Test Point





Block Diagram





YES





YES





NO





NO





NO





NO





NO





YES





YES





YES





YES





Does it work properly?





Does it work properly?





Logic level at PWR_HOLD(R201) 


of U201=High ?





Check the voltage of 


the following port at U201


+1.4V_MSMC


+1.8V_MSMP1


+2.6V_MSMP2


+2.6V_MSMA


+2.8V_TCXO





Push power-on key and check


 the level change of 


U201 PWR_ON





Check the contact of power-key


or dome switch





Replace the main board.





The Phone will power on.





The power-on procedure is completed.


The problem may be elsewhere.





Re-download software & try again





Replace U201





Charge of Change Battery and try again





NO





Check Battery Voltage


> 3.4V ?





YES





START





Checking Flow





+1.8V_MSMP1





+1.4V_MSMC











+2.6V_MSMA





+2.6V_MSMP2





+2.8V_LCD





+2.9V_RX1





+2.9V_TX





+2.9V_RX0





Test Point





4.3.1 Power





4.3.1.1 Power-On Trouble





Circuit Diagram	Test Points





� EMBED Unknown  ���





4.3 Logic Part Trouble



















































































































































































































































































































































































































































































RF OUT





Graph 4.2.1.10





Yes





Resolder CON100





RF IN





Yes





Replace


U703





NO





RF_OUT	RF_OUT





RF_IN





U100





4.2.2.9. Check Mobile Switch





Replace U100





YES


Similar?	Mobile Switch is OK See next page





Check U100 Pin1, 2


RF_IN RF_OUT


Refer to Graph4.2.2.10





Start





Test Point	Checking Flow





RF_OUT





Test Points





Checking Flow





Graph4.2.2.9





RF_IN





Circuit Diagram	Waveform





NO





Replace U101





U101





Similar?





Triplexer Circuit is OK See next page





PCS PORT





Check U101 Pin5,2


PCS_PORT ANT_PORT


Refer to Graph4.2.2.9





ANT PORT





4.2.2.8. Check Triplexer Circuit





Start





Test Point	Checking Flow





Graph4.2.2.8





Check the soldering at CON100





Circuit Diagram	Waveform








TX_PORT               





Circuit Diagram	Waveform





DP101





4.2.2.7. Check PCS Duplexer





Replace DP101





YES


Similar?	Duplexer is OK See next page





Check DP101 Pin1,3


TX_PORT ANT_PORT


Refer to Graph4.2.2.8





Start





NO





NOYes





Test Point	Checking Flow





Waveform





RF_IN





RF_OUT





Graph 4.2.2.6





Circuit Diagram





YES





Replace U107





Check U107 Pin4





NO





Check PA_R1 Signal Line





≤1.8V





≥2.70V





NO





Check +2.9V_TX Line


And PA_ON2





YES





Check U107 Pin1





YES





NO





Check VCC Line





≥3.8V





Resolder


R124,R125,R126





Resolder


U703





Check the voltage of C101


+3.1V_CTP





Check U107 Pin10





YES





PAM Circuit is OK





Similar?





Check U107 Pin2, 12


RF_IN / RF_OUT Refer to Graph4.2.2.6





NO





U112





4.2.2.6. Check PCS PAM Circuit





Checking Flow





Test Point	





Start





Yes





RX_PORT               





RF_OUT





Graph4.2.2.5





Replace F103





NO





Circuit Diagram





START





RF_IN





Circuit Diagram	Waveform





4.2.2.5. Check PCS RF TX SAW





F103





RFIN	





Test Point	Checking Flow





Start





YES


Similar?	RF TX SAW is OK See next page





Check F103 Pin1, 3


RF_IN RF_OUT


Refer to Graph4.2.2.5








RX_Q_N





RX_I_P





SBCK





SBCK





SBST





SBDT





RX_I_N





RX_Q_P





Waveform





Graph 4.2.2.4(D)





Graph 4.2.2.4(A)	Graph 4.2.2.4(C)





Graph 4.2.2.4(B)





+2.6V_MSMP PIN 32


+2.9V_TX 


PIN : 6,9,13,14,17,19,20,22,24,25,31





Chaecking Flow





Similar?





YES





YES





Check SBI Signal


Line&MSM





Check TCXO Circuit





Similar?





Check U110 Pin21


PCSB_OUT


Refer to Graph4.2.2.4 (A)





Start





RFT Circuit is OK See next page





Similar?





YES





Replace U110





Check TX_ON Signal Line&MSM





NO





≥2.5V





YES





Check U110 Pin1


(TX_ON Signal)





Check U110 Pin2, 3,4


SBCK0 SBDT0


SBST0


Refer to Graph4.2.2.4 (D)





Check Regulator Circuit





NO





Check U110 Pin1, 12,24


+2.6V_MSMP2


+2.9V_SYNTH

















OK?





NO





NO


Similar?	Check U400 MSM








YES





YES





Check U110 Pin15


TCXO


Refer to Graph4.2.2.4(C)





NO





Check U110 I/Q Data


Pin27. 28, 29,30


Refer to Graph4.2.2.4 (B)





YES





RF_OUT





NOYes





Yes





Check the signal of Following points R124.R125.R126





NOYes





NOYes














Graph 4.2.2.2








Check the soldering U703





Test Point 





Yes








Waveform





YES





Replace X700





Check U702





NO





+2.8V?





Check X700 Pin4





NO





YES


Similar?	      VCTCXO Circuit OK 





Check X700 Pin3


Refer to Graph4.2.1.2





Start





X101





+2.8V_TCXO





PIN 3 TCXO_OUT





4.2.2.2. Check VCTCXO Circuit





Test Point 





Checking Flow





LDO Circuit is OK See Next Page





	Check U701 Pin3





NO





Replace U701





+2.8V








	Start





<+2.8V_TCXO>	





Circuit Diagram





Checking Flow





+2.9V_TCXO OUTPUT





TCXO_EN





Pin3 +2.8V_TCXO





+VPWR





Pin 1 +VPWR





U701





PIN 4 DC





TEST POINT





4.2.2.1. Check Regulator Circuit





5 Check


Dual PAM Circuit








4. Check


RF SAW





Re-download SW, CAL











3. Check


RFT6150 Circuit





8. Check


Mobile Switch











Check the signal of Following points 


R106, R107, R123





2. Check


VCTCXO Circuit





7. Check


Triplexer











1. Check


LDO Circuit





6. Check


Duplexer








YES





HDET





RF OUT





START





√ Press “MENU” + “0000000” in Phone idle state


√ Press “8” FCC Test


√ Press “2” CDMA FCC


-Set CH384 & AGC 360


√ Spectrum analyzer setting


√ Oscilloscope setting





Checking Flow





Mobile SW





Triplexer





PCS duplexer





DUAL PAM





RF SAW





RFT6150








4.2.2 PCS TX Trouble





Test Point





RF IN





RF IN





RF OUT





Circuit Diagram	Waveform











Check U100  Pin 1,2 


RF_IN,


RF_OUT


 Refer to Graph 4.2.1.10





RF_OUT





RF IN











Graph 4.2.1.9





ANT_Port





CELL_Port





C100





Yes


















































Check U101 Pin 7,2 


Cell_Port,


Ant_Port


 Refer to Graph 4.2.1.9





CELL_Port





ANT_Port



































ANT_Port





C101





R124,R125,R126











TX_Port





DP101





4.2.1.7. Check DCN Duplexer





Replace DP100





YES


Similar?	Duplexer is OK See next page





Check DP100 Pin1,6


TX_PORT ANT_PORT


Refer to Graph4.2.1.8





Start





NO





Test Point	Checking Flow





TX_PORT





ANT_PORT





RF_OUT





Graph 4.2.1.6





RF_IN





Waveform





Circuit Diagram





U112





VREF_CELL





PA_R1





RF_OUTPUT





VCC1





RF_IN





4.2.1.6. Check DCN PAM Circuit





Replace U107





YES





NO


≤1.8V	Check PA_R1 Signal Line





Check U107 Pin4





YES





Check +2.9V_TX Line


And PA_ON





NO





≥2.70V





YES





Check U107 Pin6





Check U101 Pin 7





Check U107 Pin1





NO








YES


Similar?	PAM Circuit is OK 





Check U107 Pin5, 8


RF_IN / RF_OUT Refer to Graph4.2.1.6





Start





Test Point	Checking Flow





Graph  4.1.2.5





RF_OUT





RF_IN





R106





R107





Circuit Diagram	Waveform





R123





F102





4.2.1.5. Check DCN RF TX SAW





Replace F105





YES


Similar?	RF TX SAW is OK See next page





Check F102 Pin1, 3


RF_IN RF_OUT


Refer to Graph4.2.1.5





Start





NO





RF_OUT





+2.6V_MSMP PIN 32


+2.9V_TX PIN: 6,9,13,14,17,19,20,


22,24,25,31





Checking Flow





YES





YES





YES





NO








Check U201 MSM














NO





OK





Similar?





Check SBI Signal


LINE&MSM








Replace U104








Similar?	





YES





Check U104 Pin2, 3,4


SBCK0 SBDT0 SBST0


Refer to Graph4.2.1.4 (D)





Check TCXO Circuit





NO





Similar?





Check U104 Pin15


TCXO


Refer to Graph4.2.1.4(C)





Check TX_ON Signal Line &MSM





NO





≥2.5V





Check U104 Pin1


(TX_ON Signal)





Check LDO Circuit





NO





Check U104 PWR


(+2.6V_MSMP


+2.9V_TX)








Check U104 I/Q Data


Pin27. 28, 29, 30


Refer to Graph4.2.1.4 (B)





YES





RFT Circuit is OK See next page





Similar?





Check U104 Pin23


CELL_OUT


Refer to Graph4.2.1.4 (A)





Start





NO





TX_ON


 SBCK0


 SBDT0


 SBST0











U110





Check U101 Pin 7





TX_QP








TX_QM








TX_IP








TX_IM








PIN15





PIN 27


PIN 28


PIN 29


PIN 30





Replace U100





4.2.1.4. Check RFT Circuit





CELL_OUTT





Graph.4.2.1.2





TCXO_OUT





+2.8V_TCXO





4.2.1.2. Check VCTCXO Circuit





X700


T





PIN 3





PIN 3 TCXO OUT





YES





Replace X700





Check U201





NO





+2.8V?





Check X700 Pin4





NO





YES


Similar?	     VCTCXO Circuit OK See next page





Check X700Pin3


Refer to Graph4.2.1.2





Start





+2.9V_TCXO OUTPUT





TCXO_EN





+VPWR





TCXO_EN





PIN 1


T





PIN 1 :Vpwr


T





PIN 3:Vout


T





+VPWR





PIN 6


T





U403


T





RFT6100





VCTCXO





Mobile SW





Triplexer





DCN duplexer





DUAL PAM





DCN TX SAW





RFT6150





PCS TX SAW





Graph 4.1.2(h)





Circuit Diagram





Graph 4.1.2(i)





RX0_IM	RX_Q_N





RX0_QP





RX0_QM	RX_Q_N





RX0_IP





Waveform





Check Pin7  (RX0_QM)


      Pin8  (RX0_QP)


      Pin9  (RX0_IM)


      Pin10 (RX0_IP) 


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.2(h,i)








All PCS RX check is completed if the phone still does not work, change the board.





Replace Board





No





Similar?








Yes











Replace U105





No











Similar?





Yes








Re-download the


Software & Calibrate














Checking Flow 





Check Pin7  (RX0_QM)


      Pin8  (RX0_QP)


      Pin9  (RX0_IM)


      Pin10 (RX0_IP) 


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.2(h,i)








U105  Pin7  (RX0_QM)


       Pin8  (RX0_QP)


       Pin9  (RX0_IM)


       Pin10 (RX0_IP)





4.1.2.5 Checking Rx I/Q data





Test Point





Circuit Diagram





DP100 Pin 2





Graph 4.1.2(f)





Graph 4.1.2(g)	





U106 Pin 5





Graph 4.1.2(e)





U107 Pin 2





Waveform








RF Signal Path is OK. See next Page to check RX I/Q data signal.





Detected Signal?





No





Yes





Check C1128





Changing DP100





No





Yes





Yes





Detected Signal?





Detected Signal?





Check DP101 Pin 2


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.2(g)








Check 


L122





U101





Detected Signal?





No





No





Yes





� EMBED Unknown  ���





Yes





Check U101 Pin 5


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.2(f)








Changing U101





DP100





L122





C100





Detected Signal?





Detected Signal?





Check U100 Pin 2


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.2(e)








Yes





No





No





Changing U100








Check C100





4.1.2.4 Checking RF Signal path (Mobile S/W, Triplexer, Duplexer)





U100





DP101





Checking Flow





Test Point





Circuit Diagram








Graph 4.1.2(c)	Graph 4.1.2(d)





SBDT0





Waveform








SBCK0





SBST0





SBCK0





Check Pin 49(SBCK0),


Pin 50(SBST0)


Pin 51(SBDT0) of U105


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.2(c,d)











Check Pin 49(SBCK0),


Pin 50(SBST0)


Pin 51(SBDT0) of U105


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.2(c,d)











Control Signal is Ok


See next Page to Check


RF signal path





YES





Similar?





NO








Replace Board





Control Signal is Ok


See next Page to Check


RF signal path





YES





Similar?





NO





Download the SW





Checking Flow 





U105  Pin49 (SBST0)


       Pin50 (SBDT0)


       Pin51 (SBCK0)





4.1.2.3 Checking Control signal





Test Point





Circuit Diagram








Waveform








Graph 4.1.2(a)





Graph 4.1.2(b)





SW Download


Replace Board





NO





X101. Pin3





X101. Pin4





Changing X101


Check C116, C119


C118, R102





Similar?





YES





VCTCXO Circuit is OK. See next Page to check Control signal.





Check X101 Pin 3


◆ Refer to Graph 4.1.2(a)





Check X101 Pin 4


◆ Refer to Graph 4.1.2(b))





NO





The Problem may be Logic part


Refer to Logic troubleshoot





+2.8V OK?








YES





19.2MHz?








No





4.1.2.2 Checking VCTCXO Circuit





Test Point





Checking Flow (VCTCXO)





VCTCXO Circuit is OK .See


 next Page to check Control signal.





YES





Circuit Diagram








DC Power supply Circuit


is OK. See next Page to


check VCTCXO Circuit.





Yes





No





�





�





+2.9V_RX1 is OK?








Check R221


of U201








Check L203


of U201








Yes





No





PIN 6:chip enable


T





+2.85V_BT





+3.0V_T_Flash





PIN 4 DC





+2.9V_RX1 is OK?








The Problem may be Logic part


Refer to Logic troubleshoot








+2.6V_MSMP2 is OK?





Check L203


of U201








Yes





No





Test Point





Checking Flow





4.1.2.1 Checking DC Power supply Circuit (PMIC)





Re-download SW, CAL





2. Check


 VCTCXO Circuit





5. Check


Rx I/Q data





1. Check


DC Power supply circuit





4. Check


RF Signal path





3. Check


Control signal





Rx TEST SETUP (HHP)


- Test Channel: 600


- Test Band: US PCS


- SID:	4119


- Sector Power: -30 dBm Spectrum Analyzer Setting Oscilloscope Setting





START





Checking Flow





dsdfsdfsdfddddddddMobile SW








Triplxer





dsdfsdfsdfddddddddMobile SW








Mobile SW





dsdfsdfsdfddddddddMobile SW














Test Point





4.1.2 PCS RX





Graph 4.1.1(h)





Circuit Diagram





Graph 4.1.1(i)





RX0_IM	RX_Q_N





RX0_QP





RX0_QM	RX_Q_N





RX0_IP





Waveform





Check Pin7  (RX0_QM)


      Pin8  (RX0_QP)


      Pin9  (RX0_IM)


      Pin10 (RX0_IP) 


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.1(h,i)








All DCN RX check is completed if the phone still does not work, change the board.





Replace Board





No





Similar?








Yes











Replace U105





No





Similar?





Yes








Re-download the


Software & Calibrate














Check Pin7  (RX0_QM)


      Pin8  (RX0_QP)


      Pin9  (RX0_IM)


      Pin10 (RX0_IP) 


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.1(h,i)








Checking Flow 





U105  Pin7  (RX0_QM)


       Pin8  (RX0_QP)


       Pin9  (RX0_IM)


       Pin10 (RX0_IP)





4.1.1.5 Checking Rx I/Q data





Test Point





Circuit Diagram





Graph 4.1.1(g)	





Graph 4.1.1(f)	





Pin7





U106 Pin 7





Graph 4.1.1(e)





U107 Pin 2





Waveform








RF Signal Path is OK. See next Page to check RX I/Q data signal.





Check 


C127 & L124


C1127





Detected Signal?





Yes





No





Changing DP101





Detected Signal?





Detected Signal?





Check DP101 Pin 3


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.1(g)








Yes





Yes





No





C127,   L124 





Detected Signal?








Check C185&L121








Changing U106





C1128





Check U106 Pin 7


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.1(f)








Yes





Yes





No





No





U101. Pin7





DP101. Pin3





Detected Signal?





Detected Signal?





Check U107 Pin 2


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.1(e)








Changing U107








Check C101





4.1.1.4 Checking RF Signal path (Mobile S/W, Triplexer, Duplexer)





dsdfsdfsdfddddddddMobile SW








Yes





No





No





U107. Pin2





Checking Flow





Test Point





Circuit Diagram








Graph 4.1.1(c)	Graph 4.1.1(d)





SBDT0





Waveform








SBCK0





SBST0





SBCK0





YES





Control Signal is Ok


See next Page to Check


RF signal path





Check Pin 49(SBCK0),


Pin 50(SBST0)


Pin 51(SBDT0) of U105


Check if there is


Any Major Difference


◆ Refer to Graph 4.1		.1(c,d)











Similar?





NO








Replace Board





Similar?





NO








Download the SW





YES





Control Signal is Ok


See next Page to Check


RF signal path





Check Pin 49(SBCK0),


Pin 50(SBST0)


Pin 51(SBDT0) of U109


Check if there is


Any Major Difference


◆ Refer to Graph 4.1.1(c,d)











Checking Flow 





U109  Pin49 (SBST0)


       Pin50 (SBDT0)


       Pin51 (SBCK0)





4.1.1.3 Checking Control signal





Test Point





Circuit Diagram








Waveform








Graph 4.1.1(a)





Graph 4.1.1(b)





SW Download


Replace Board





NO





PIN 3 TCXO OUT





X101. Pin4





Changing X700





Similar?





VCTCXO Circuit is OK. See next Page to check Control signal





YES





Check X700 Pin 3


◆ Refer to Graph 4.1.1(a)





Check X700 Pin 4


◆ Refer to Graph 4.1.1(b)





Test Point





19.2MHz?


No





+2.8V OK?








YES





NO





The Problem may be Logic part


Refer to Logic troubleshoot





4.1.1.2 Checking VCTCXO Circuit





Checking Flow (VCTCXO)





VCTCXO Circuit is OK .See


 next Page to check Control signal





YES





Circuit Diagram








The Problem may be Logic part


Refer to Logic troubleshoot








The Problem may be Logic part


Refer to Logic troubleshoot




















+2.9V_RX1 is OK?








Check BAT CONTACT








No











No





No





Yes





Yes





Yes





Check L203


of U201





Check R221


of U201





Check L203


of U201





+2.6V_MSMP2 is OK?





DC Power supply Circuit


is OK. See next Page to


check VCTCXO circuit.





+2.9V_RX1 is OK?














Test Point





Checking Flow





4.1.1.1 Checking DC Power supply Circuit (PMIC)





Redownload SW, CAL





2. Check


 VCTCXO Circuit





5. Check


Rx I/Q data





1. Check


DC Power supply circuit





4. Check


RF Signal path





3. Check


Control signal





Rx TEST SETUP (HHP)


- Test Channel: 384


- Test Band: US Cellular


- SID:	2004


- Sector Power: -30 dBm Spectrum Analyzer Setting Oscilloscope Setting





START





Checking Flow





R221





RFR6500





PCS_Duplexer





DCN_Duplexer





Triplexer





Mobile S/W





Test Point





F609 ->D2[12],D2[13],D2[14],D2[15]





F602 -> D2[4],D2[5],D2[6],D2[7]





F608 ->D2[8],D2[9],D2[10],D2[11]





F607 ->SLCD_CS,LCD_RESET/,WE2/,MLCD_CS,LCD_RS,OE2














Re Assemble Main/Sub Board





Y





Replace U201





No








     LG Electronics Inc. 
1
- 6 -

     LG Information & Communications, Ltd.  


_1231846647

_1231846954

_1231847046

_1231847232

_1231847298

_1231847326

_1231847376

_1231847313

_1231847250

_1231847143

_1231847159

_1231847068

_1231846999

_1231847020

_1231846974

_1231846791

_1231846848

_1231846897

_1231846811

_1231846765

_1231846780

_1231846668

_1231846416

_1231846612

_1231846630

_1231846484

_1209544766.vsd
       Similar?�

START�

Check U105 Pin7,2
CELL_PORT
ANT_PORT
Refer to Graph 4.2.1.9�

U105�

Check U105 Pin 7�

�

Triplexer Circuit is�

YES�

NO�

        >=2.5�

Replace U102�

Check CDMA_MODE
Signal Line & MSM�

NO�

YES�


_1231846299

_1231846389

_1231846192

_1209550292.vsd
     Similar?�

START�

Check U107 Pin1,2
RF_IN
RF_OUT
Refer to Graph 4.2.1.10�

U407�

Replace U107�

�

Mobile Switch is OK�

YES�

NO�


_1209381916.vsd
Peripheral Circuits�

PLL�

HK ADC�

Microprocessor Bus�

Address/Data�

General�

-�

Purpose Interface Bus�

JTAG Interfaces�

Rx�

ADCs�

RF�

Interface�

TX�

DACs�

AUX�

SBI�

SBI�

Vocoder�

EVRC�

13�

k�

MP4�

MIDI�

QDSP4000�

Integrated�

CODEC�

MODE Select�

Interface�

ETM�

gpsOne�

Processor�

UART1�

UART2�

UART3/USB�

ARM9�

TDMI�

Memory�

Controller�

General�

-�

Purpose�

Interface�

Camera module�

(1.3M C�

-�

MOS)�

PC�

Connectivity�

Test/Debug�

System�

Digital Test Bus�

JTAG�

Keypad input�

RFT6100�

RFR6000�

MSM6500�

Mobile Station Modem�

VCTCXO�

19.2�

MHz�

1�

2�

3�

4�

5�

6�

7�

8�

9�

*�

#�

0�

POWER �

Amplifier�

LCD�

Memory�

NAND Flash(512Mb)�

LP�

-�

SDRAM(256Mb)�

EAR         �

External�

EAR/MIC�

KEYPAD, BEZEL,�

SIIDEKEY�

MIC�

Audio AMP�

Data Comm�

DM�

USB�

USB �

Transceiver�

BATTERY�

HDET�

INNER CHG�

+2.9�

V_Rx_LDO�

DFM�

Processor�

CDMA�

Processor�

RF Controls�

2         �

Speaker �

LDO�

CAMIF�

Bluetooth�

Baseband�

Processor�

B/T Chip�

Antenna�

Bluetooth RF�

2         �

Peripheral Circuits�

PLL�

HK ADC�

Microprocessor Bus�

Address/Data�

General�

-�

Purpose Interface Bus�

JTAG Interfaces�

Rx�

ADCs�

RF�

Interface�

TX�

DACs�

AUX�

SBI�

SBI�

Vocoder�

EVRC�

13�

k�

MP4�

MIDI�

QDSP4000�

Integrated�

CODEC�

MODE Select�

Interface�

ETM�

gpsOne�

Processor�

UART1�

UART2�

UART3/USB�

ARM9�

TDMI�

Memory�

Controller�

General�

-�

Purpose�

Interface�

Camera module�

(1.3M C�

-�

MOS)�

PC�

Connectivity�

Test/Debug�

System�

Digital Test Bus�

JTAG�

Keypad input�

RFT6150�

RFR6500�

MSM6500�

Mobile Station Modem�

VCTCXO�

19.2�

MHz�

1�

2�

3�

4�

5�

6�

7�

8�

9�

*�

#�

0�

POWER �

Amplifier�

LCD�

Memory�

NAND Flash(512Mb)�

LP�

-�

SDRAM(512Mb)�

EAR         �

External�

EAR/MIC�

KEYPAD, BEZEL,�

SIIDEKEY�

MIC�

Audio AMP�

Data Comm�

DM�

USB�

USB �

Transceiver�

BATTERY�

HDET�

INNER CHG�

+2.9�

V_Rx_LDO�

DFM�

Processor�

CDMA�

Processor�

RF Controls�

2         �

Speaker �

LDO�

CAMIF�

Bluetooth�

Baseband�

Processor�

B/T Chip�

Antenna�

Bluetooth RF�

2         �


_1209539103.vsd
  1.Check     
    LDO Circuit�

START�

v Press �MENU� + �0000000� in Phone idle state
v Press �8� FCC Test
v Press �2� CDMA FCC
-Set CH384 & AGC 360
v Spectrum analyzer setting
v Oscilloscope setting�

2.Check VCTCXO
  Circuit�

3.Check RFT6150
   Check�

4. Check RF SAW�

5.Check Dual 
    PAM�

6.Check Duplexer�

7.Check Triplexer�

8.Check Mobile
   Switch�

Re-download, CAL�


_1185188377.doc















_1208758959.vsd
Peripheral Circuits�

PLL�

HK ADC�

Microprocessor Bus�

Address/Data�

General�

-�

Purpose Interface Bus�

JTAG Interfaces�

Rx�

ADCs�

RF�

Interface�

TX�

DACs�

AUX�

SBI�

SBI�

Vocoder�

EVRC�

13�

k�

MP4�

MIDI�

QDSP4000�

Integrated�

CODEC�

MODE Select�

Interface�

ETM�

gpsOne�

Processor�

UART1�

UART2�

UART3/USB�

ARM9�

TDMI�

Memory�

Controller�

General�

-�

Purpose�

Interface�

Camera module�

(1.3M C�

-�

MOS)�

PC�

Connectivity�

Test/Debug�

System�

Digital Test Bus�

JTAG�

Keypad input�

RFT6100�

RFR6000�

MSM6500�

Mobile Station Modem�

VCTCXO�

19.2�

MHz�

1�

2�

3�

4�

5�

6�

7�

8�

9�

*�

#�

0�

POWER �

Amplifier�

LCD�

Memory�

NAND Flash(512Mb)�

LP�

-�

SDRAM(256Mb)�

EAR         �

External�

EAR/MIC�

KEYPAD, BEZEL,�

SIIDEKEY�

MIC�

Audio AMP�

Data Comm�

DM�

USB�

USB �

Transceiver�

BATTERY�

HDET�

INNER CHG�

+2.9�

V_Rx_LDO�

DFM�

Processor�

CDMA�

Processor�

RF Controls�

2         �

Speaker �

LDO�

CAMIF�

Bluetooth�

Baseband�

Processor�

B/T Chip�

Antenna�

Bluetooth RF�

2         �

Peripheral Circuits�

PLL�

HK ADC�

Microprocessor Bus�

Address/Data�

General�

-�

Purpose Interface Bus�

JTAG Interfaces�

Rx�

ADCs�

RF�

Interface�

TX�

DACs�

AUX�

SBI�

SBI�

Vocoder�

EVRC�

13�

k�

MP4�

MIDI�

QDSP4000�

Integrated�

CODEC�

MODE Select�

Interface�

ETM�

gpsOne�

Processor�

UART1�

UART2�

UART3/USB�

ARM9�

TDMI�

Memory�

Controller�

General�

-�

Purpose�

Interface�

Camera module�

(1.3M C�

-�

MOS)�

PC�

Connectivity�

Test/Debug�

System�

Digital Test Bus�

JTAG�

Keypad input�

RFT6150�

RFR6500�

MSM6500�

Mobile Station Modem�

VCTCXO�

19.2�

MHz�

1�

2�

3�

4�

5�

6�

7�

8�

9�

*�

#�

0�

POWER �

Amplifier�

LCD�

Memory�

NAND Flash(512Mb)�

LP�

-�

SDRAM(512Mb)�

EAR         �

External�

EAR/MIC�

KEYPAD, BEZEL,�

SIIDEKEY�

MIC�

Audio AMP�

Data Comm�

DM�

USB�

USB �

Transceiver�

BATTERY�

HDET�

INNER CHG�

+2.9�

V_Rx_LDO�

DFM�

Processor�

CDMA�

Processor�

RF Controls�

2         �

Speaker �

LDO�

CAMIF�

Bluetooth�

Baseband�

Processor�

B/T Chip�

Antenna�

Bluetooth RF�

2         �


_938585124.doc















